==============================================================

Beep MIDI Player LXMIDI ver. 1.71

Copyright (C) 1996,1998 HARUYA

==============================================================

1.Introduction

LXMIDI is a software MIDI synthesizer developed for the HP

100LX/200LX.  It conducts a real-time conversion from MIDI

data (in Standard MIDI File Format 0) into 8- to 16-part

polyphonic multitimbral square-wave PCM data and plays it with

the built-in speaker of the 100LX/200LX.

Although wave forms and the number of parts (timbres) are

inferior to normal MIDI sound sources, the implementation of

important MIDI events (by GM standard), such as the percussion

set, pitch modulator, pitch bend, portamento and others,

enables high fidelity to the original music, if it has

relatively low number of voices.  Reverb effect is applied to

help adding a depth to the sound.

It can be used also on IBM-PC compatibles or on NEC PC-98.

Normal speed 100/200LX can be used.  However my recommendation

is to use a machine with double-speed crystal upgrade (or

faster) to eliminate high pitch noise and to enable largest

polyphonic notes, for the sound with higher quality.

Remark:

This program makes full utilization of high speed hardware

interrupts for parallel processing of synthesis and playback

of PCM data.  Therefore, collision with TSR programs,

playback of corrupted MIDI data or small bugs in the program

can easily make the system hang. Although this version is

more stable than the initial ones, I do recommend you to

back-up C: drive and to make other precautions before trying

to run this program.

2. Quick Start

Please prepare MIDI data in Standard MIDI File Format 0.

From DOS prompt, enter:

LXMIDI -N1 -X -O8 -C120 SAMPLE.MID (normal machine)

or

LXMIDI -N1 -X SAMPLE.MID           (double speed machine)

(SAMPLE.MID is assumed to be the name of your MIDI file).  Now

our 100/200LX is reborn as a palmtop MIDI sound module.

If you are a user of a normal speed machine, you will notice

the high pitch noise.  Reducing the value of the '-C' option

may improve the sound. First reduce the value to a point

where you encounter a skipping sound, then increase the value

by 3 to 5.

If you want to play the MIDI date in formats other than SMF

Format 0, please refer to Section 5 below.

Your should have at least 130 KB of the conventional memory

(or 70 KB, if EMS is available) to run LXMIDI.  If your sound

data are larger, you need more. If enough memory is not

available, LXMIDI will terminate during reading from the file.

Specifying '-S' option, when more than 4 pages of EMS is

available, will use EMS and thus enables the playback of

larger data.

From Ver 1.70,  LXMIDI can create and play music data in

Object File format (extension .LXM), an internal data format

of LXMIDI. Running LXMIDI with '-W' will create the LXM data

file.  Although larger in file size in most cases, it can be

loaded faster and enables to start the music immediately.

******* Playing an LXM file created by earlier **************

*       versions of LXMIDI                                  *

*                                                           *

* This version of LXMIDI can play an LXM file created by    *

* earlier version of LXMIDI without any problems.  However, *

* the corrections added to this version, such as            *

* the elimination of excess sound volume of certain         *

* instruments, is not implemented in the playback, which    *

* stays the same as done by previous versions.  Create the  *

* LXM files again to reflect the modification added to this *

* version.                                                  *

*                                                           *

*************************************************************

3. Command reference

Usage of LXMIDI is as follows:

LXMIDI [option(s)] <filename>[.MID]

<filename>

The name of the MIDI data file to play. Default

extension is '.MID'.

option -Nn

n = 0  (default)  HP100/200LX with 36 MHz crystal (so called

2.27-times speed upgrade)  (1)

n = 1  HP100LX/200LX with normal speed or double speed

crystal.  '-C' option is also necessary for normal

speed.

n = 2  for IBM-PC compatible. (2)

n = 3  HP100/200LX with 32.0 MHz crystal (3)

n = 4  HP100/200LX with 40.0 MHz crystal and 31/32 MHz driver

n = 9  for NEC PC-98 and its compatible.

Notes:

(1) The reason that -N0 is default is because -N1

will hang the 36 MHz machine.

(2) Under normal condition, temporary patches are

applied to interrupt routines specific to LX for

better sound quality.  -N2 option disables this

patch so LXMIDI can run on IBM-PC compatibles.

(3) Option -N1 also works for 32 MHz crystal. However

tempo and pitch would be higher by approximate 1%.

Option -N3 is recommended.

Option -W

This option create an Object File (LXM file), without

playing the music data. You can play the object file

by running LXMIDI with its filename specified. It will

drastically reduce the loading time.

When you play an Object File, the options -O,-M,-P,-V,

-A,-C,-T,-R,-D are ignored. (The option values you

used at the creation of the Object File are applied).

Option -On

This option specifies maximum polyphonic notes. The

parameter n should be 4, 8, 12 or 16.  Default is 12.

Larger value requires larger CPU power.  If your

crystal is normal speed, setting of '-O8' together

with '-X' option (see below) is recommended.

Option -X

This option disables LCD display while the music is

played.  Its effect is as follows.

o CPU power is increased by more than 7 %, since LCD

controller is stopped and CPU gains control of the

bus all the time.  You can specify better value to

-O and -C.

o With the same reason, PCM interrupts by CPU is more

stable, which leads to less jittering noise during

playback.

o Power consumption is decreased.

I recommend to use this option whenever possible.

This option does not work with -N2 or -N9.

Option -S

Enables the usage of EMS.  You can play large size

data which cannot be played with the conventional

memory. If you have no room in conventional memory

(it is the case, when you run LXMIDI from the System

Manager), you can use this option, which consumes as

little as 70 KB of conventional memory.

Option -Mccc...ccc       c:{0123456789ABCDEF}

The parameter specifies MIDI channels to be muted

(masked).  For example:

-M128AC

Normal MIDI sound source (GM or GS compliant) has 16

to 64 channel multi-timbral/polyphonic capability.

Therefore some of the sound data requires voices far

exceeding 16 notes available on LXMIDI.

Even if the data requires parts exceeding this

number, LXMIDI tries to conserve the original music,

but may not success, if the number of voices are too

many. In that case, masking of some MIDI channels and

reducing polyphonic sounds may help.

Percussion sound source implemented by LXMIDI does not

have very good quality, due to the limitation of the

machine power.  If you do not like this percussion,

simply mask its part by '-M9' option.

Option -P

This option ignores the instrument given by MIDI data.

All the sounds are then made with a flat envelope.

LXMIDI is a synthesizer with square-wave sound source

and cannot change wave form to recreate the sound

tone of an instrument.  Instead, LXMIDI uses the

change of envelope to recreate the sound.  (according

to instruments as assigned by GM standard)  This

option suppress this function.

Option -Vn

This option sets the sound volume (0 to 100).  If it

is set to '0', LXMIDI automatically sets the volume

to the optimum value.  The default setting is '0'

(automatic volume control).  Value too high will

distort the sound if MIDI data contain high-volume

part.  This setting is independent from the system

volume set by Setup menu of the 100/200LX.

Option -An

This option sets sound volume under the automatic

volume control (10 to 200 %).  If you use the

automatic volume control, LXMIDI selects the optimum

volume level, the maximum value with no distortion.

If you want to have larger volume and tolerate the

distortion at laud parts, then you may set this

option to a value exceeding 100.

Option -Cn

Sets cycle duration of PCM (40 to 255).  Default is 64

for-N1 and -N2 options; 95 for -N0 option.  Low value

eliminates the high-pitch noise but requires higher

CPU power.  Too low a value (which exceeds the CPU

power) will make the sound skip.

You should increase this setting, if you use normal

speed 100/200LX.  First set a large amount (around

120) and gradually decrease it.  The setting should be

a little larger than the value where the playback

starts to suffer from skipping sound.

Option -K

With this option, LXMIDI will wait for you to press

any key before it starts playing.

Option -Tn

This option set tempo (10 to 1000) in per cent unit.

Examples: -T103 sets tempo as 103% of the original

-T50  sets tempo as 50% of the original

-T200 sets tempo as 200% of the original

Option -Rn

Transpose (pitch shift) value is specified (-24 to 24)

in the unit of half step (a twelfth octave).

Examples: -R1   raise by half step.

-R12  raise by an octave.

-R-12 lower by an octave.

Option -Dn

This option sets the volume of percussion part

relative to other parts (in per cent unit).

Examples:

-D100  default.

-D200  volume of percussion is twice as large.

-D50   volume of percussion is reduced to half.

Option -En

Reverb effect level is adjusted by this option (1 to

6).

Examples: -E1   -6 dB (maximum)

-E2   -12 dB (default)

-E6   -36 dB (minimum)

4. Key input during playback

LXMIDI first analyses the input MIDI file and creates Object

Data (internal data for playback).  During this period, pressing

any key can stop the execution.

After the analysis, the music starts (If -K option is used,

LXMIDI waits for a key before starting).  If -X option is

specified, the LCD display will be turned off. If you run

LXMIDI with an Object File, the music start immediately.

Following keystrokes is recognized after the music starts.

[SHIFT]  Fast playback. The music is played back with double

speed while this key is pressed down.  Since the

[SHIFT] key of the HP100/200LX is a toggle key

(sticky), pressing it starts the fast playback and

pressing it again returns it back to normal

playback.

[ALT]    This key will pause the music temporarily.  You can

switch the power off or on during the pause.  Press

any key to resume the playback.

[CTRL]   These key can interrupt the playback and exit.

Do not use [Ctrl]+[C] or [Ctrl]+[Break], as it may cause

problems such as the non-released EMS.  Always use [CTRL] key

to exit.

5. MIDI Data playable by LXMIDI

LXMIDI supports MIDI data in SMF (Standard MIDI File) Format 0,

with a file extension of .MID.  Data in SMF format 1 can be

converted to SMF format 0, by using following utilities.

MIDI1TO0.EXE     SMF(.MID) format 1 to format 0 Conversion

MD1T0V04.LZH  NIFTY SERVE FMIDITOL forum LIB.8 #15

SMF1TO0.EXE      SMF(.MID) format 1 to format 0 Conversion

SMF10V10.LZH  NIFTY SERVE FMIDITOL forum LIB.8 #25

(SMF10V11.LZH)

File extension .STD is also used for SMF format.

Popular formats such as RCP, MCP, MM2, MMC, SEQ or SNG can be

converted to SMF by following utilities.

CVS.EXE          .MCP/.RCP      to SMF(.MID) format 1

CVS506.LZH    NIFTY SERVE, FMIDITOL forum, LIB.8 #69

MM2S.EXE         .MM2/.MMC/.SEQ to SMF(.MID) format 0/1

MM2S31.LZH    NIFTY SERVE, FMIDITOL forum, LIB.8 #30

SNG2S.EXE        .SNG           to SMF(.MID) format 0/1

SNG2S33.LZH   NIFTY SERVE, FMIDITOL forum, LIB.8 #31

I would like to express my thanks to the authors of these

valuable utility programs.

( Translator's Note:

These software are all available on NIFTY SERVE, a Japanese

information service, a licensee of CompuServe, and meant

to be for Japanese users.  I suppose similar conversion

utilities can be found on MIDI forums of Compuserve.

MIDI1TO0.EXE, one of the above utilities,  can be downloaded

from Compuserve, HPHAND forum, LIB 11, as MIDI1TO0.ZIP or

from www.palmtop.net/super.html. )

If you need, other software are also available at following

web sites:

ftp://www.vector.co.jp

http://www.vector.co.jp

6. MIDI events supported by LXMIDI

Following is the MIDI events supported by LXMIDI, described

in MIDI Implementation Chart format.

You can skip this section, if you are not familiar with this

description.  You can use LXMIDI without the knowledge

described in this section.

FUNCTION           RECEIVE       NOTES

----------------------------------------------------------------

BASIC CHANNEL      1-16

MODE               X 3           16-POLYPHONIC 16-MULTI TIMBRE

NOTE NUMBER        5-126

VELOCITY NOTE ON   O 9n,V=1-127

NOTE OFF  X

AFTER TOUCH        X

PITCH BENDER       O             PRECISION:25CENT.(1/4 CHROMATIC SCALE)

CONTROL CHANGE  0  O             BANK SELECT(MSB) **

1  O             PITCH MODULATION DEPTH

5  O             PORTAMENTO TIME

6  O             DATA ENTRY

7  O             VOLUME

11  O             EXPRESSION

64  O             HOLD 1 (DUMPER PEDAL)

65  O             PORTAMENTO ON/OFF

100,101 O             RPN LSB,MSB *

120 O             ALL NOTE OFF

121 O             CHANNEL RESET

122-127 O             ( = ALL NOTE OFF)

PROGRAM CHANGE     O 0-121       SQUARE WAVE

ENVELOPE MODELING TIMBRE ***

EXCLUSIVE          O             UNIVERSAL MASTER VOLUME

GS MASTER VOLUME

GS USE FOR RHYTHM PART

COMMON(SONG,TUNE)  X

REALTIME

(CLOCK,COMMAND) X

---------------------------------------------------------------

*   MSB=0,LSB=0 (BENDER RANGE) ONLY.

**  BANK 126,127 ONLY.

*** PROGRAM NUMBER IS BASED ON GENERAL MIDI STANDARD.

CH.10:SUPPORT GM STANDARD PERCUSSION SET ONLY.

7. Revision History

Ver 1.10

o First Release

Ver 1.20 (not released)

o Percussion set implemented.

o Resolving process for polyphonic sound was improved for

higher fidelity.

o Reverb effect introduced.

o Playback routine optimized, with 15% gain in speed.

o Default extension of '.MID' is applied.

o Removal of a lot of small bugs.

Ver 1.21

o Removed a bug that expression and pitch bender/portamento

events cannot run simultaneously.

o '-T' and '-R' options added.

Ver 1.22

o Percussion data set was tuned and improved (I hope).

o Resolving process for exceeding polyphonic sound are

modified for (probably) better reproduction of the

original.

o '-D' and '-E' options added.

o Several bugs removed.

Ver 1.23 (not released)

o Resolving process for exceeding polyphonic sound are

improved.

o Removed a bug that running status in not adequately

handled after the execution of a meta event.

Ver 1.30

o Removed a bug that caused hangup after the completion of

playback.  (I would like to thank to Chikara and all the

other people who reported or commented on this problem.)

o Added the automatic volume control, which sets the volume

to the maximum without distortion of the sound.

o [Shift] key introduced as a pause key for the playback.

o PITCH MODULATOR implemented.  Vibrato is enabled.

o Clock adjustment after the playback now implemented.

o Removed plural bugs.

Ver 1.40

o Expanded polyphonic notes to max 16.

o Added '-X' option, which disables LCD display while

playback for increased CPU power and better sound

quality.

o Optimization of playback routines.  Introduction of the

self modification method for higher speed (increase by

18%).  12- or 16-note polyphony with no '-X' option is

now practical on machines with double-speed crystal or

higher.  Smaller value in -C option is enabled on a

normal speed machine with 8-note polyphony.

o Changed algorithm for the automatic volume control.

o Now runs on NEC PC-98.

o Sound volume balance is adjusted for each timbre.

o Removed a bug by which LXMIDI did not run on IBM-PC.

o Added '-A' option.

o Compiling model is changed to tiny model (.COM format)

to reduce the memory necessary for execution by 30 KB.

Ver 1.41

o Removed a bug that the date is not renewed if a new date

arrives during playback.

o Removed a bug that the time is not correctly adjusted

when playback is done at 19 to 20 o'clock.

Ver 1.42

o 4-note polyphonic is supported.

Ver 1.50

o EMS is supported by '-S' option.

o Fixed the low beep tone problem after the playback.

Ver 1.51

o Fixed the bug that the clock adjustment does not work

properly.

Ver 1.52 (not released)

o Fixed the bug that large data will hang the system

without any 'Memory Full' error message.

Ver 1.60

o Release portion is added the envelop.

Ver 1.61

o Reviewed the tones of the percussion set for improvement.

o Fixed the incorrect pitch of some instruments (woodblock

for example).

o Fixed the bug that '-C' option is ignored if '-N' option

is specified afterward.

o Exclusive messages of GS MASTER VOLUME and GS USE FOR

RHYTHM PART are supported.  The music data for GS sound

source, which uses plural rhythm parts or uses the master

volume for fade-in/fade-out, are now played correctly.

Ver 1.62

o Too large vibrato introduced by Ver 1.61 is adjusted.

o Program Bank 126 and 127 (MT-32/CM-64 compatible

assignment) is supported.  The data using these banks

are now played correctly.

o Volume and envelope are adjusted for several instruments.

o Interval of woodblocks and drums are adjusted.

Ver 1.63

o Removed a bug (from Ver.1.61) that a tune using MIDI

event 0xF7 is not correctly played.  My thanks to Mr.

Poke (QZL00043@niftyserve.or.jp) who kindly pointed it

out to me.

Ver 1.64 (not released)

o Volume balance between sounds are adjusted.

Ver 1.70

o Full review of the sound tones with 55MAP of SC-88VL as

a target.

o Reduced popping sounds at starting and ending playback.

o Supported saving/loading of object files.

Ver 1.71

o Excessively loud instruments were adjusted to adequate

sound volume.  Envelopes and volumes of all the

instruments were reviewed.

o Eliminated the distortion caused by the too large volume

set for short music by automatic gain control.

o Introduction of fast playback.  Key assignment was changed

for this new feature.

7. Copyright and disclaimers

This program is a free software.  Its copyright is owned by its

author, HARUYA <pxw03430@nifty.ne.jp>.

Distribution is permitted, if it is not for commercial

purposes.  When you repost it to the other networks or other

forums, please notify the author (prior notification is not

necessary).  If you repost this software, you are obliged to

update it, when a new version of this software is released.

Prior notification is necessary, it this software are to be

bundled to publication - a book or a periodical.

The attached source code shall not be used, as a whole or as a

large part of it, for other software, without permission from

the author.   Distribution of this software which are modified

from the original is not permitted unless allowed by the

author.

The author will be very happy, if he is informed of any bugs

of this software.  He is more happy, if the cause of the bugs

is also reported.

The author has no responsibility for any damages, which may be

caused by use of, or inability to use, this program.  Use it

at your own risk.

8. Postscript

I started to create LXMIDI in August, 1996.  The first program

has a lot of bugs and its functions are not rich.  However, the

warm supports and bug reports from its users guided it all

through to the current version.  I would like to thank to all

the people who kindly downloaded the program and posted their

comments and requests in the forums or informed me by e-mail

messages. The feeling that the users enjoy my creation

encourages me - its author - and gives me an energy to further

improve it.

Your comments and opinions are always welcome.

March 14, 1998

HARUYA

pxw03430@nifty.ne.jp

Note:

This document is a translation of the original Japanese

document LXMIDI.DOC, included in this package as

LXMIDI.JPN.  Translated from Japanese by Toshiki Sasabe,

toshiki@j.email.ne.jp.