Upgrading a 384k Racore/Quadram memory card to 640k for a total of 736k

by Roland Fritz & Hank Kennedy

----------------------------------------------------------------------

Like other complex modifications, this is performed at your own risk.

NOTE:Although these instructions provide for adding four banks of ICs

for a total increase of 256K, you may choose to add anywhere from

1 to 4 banks.

-----------------------------------------------------------------------

Parts: 32  -  64k DRAMs  type 4164  (150 or 200 ns)

1  -  14 pin I.C. socket

4  -  30 ohm 1/4 watt resistors  5%

2  -  74HC08

1  -  74HC51  (see text)

1  -  74HC138

3 ft of small gauge solid wire, also called wire-wrap wire

Note that the logic I.C.'s used may be either 74LS or 74HC.

Note  that the board is removed from the computer for soldering.  It

can  be  carefully  lifted  straight up from the expansion box.  Gently

lift  the  right  side (the side with the TMS4500 chip) to clear the 60

pins from the bus.

This is not a job for those lacking good soldering skills.  This job

is  similar or a little greater in work content to the instructions for

upgrading  the  Microsoft  128K  sidecar to 512K.   Three to four hours

labor and good solder skills are required.

1.---------------------------------------------------------------------------

Run  memory diagnostics on the full 384k Racore/Quadram memory board

and troubleshoot until passing all tests.

Pull  all 4 banks of 8 SOCKETED 64k DRAMs and set them aside.  There

are 6 banks on the card with banks 0 & 1 soldered in.

Replace  them  with  new 64k DRAMs and again verify all memory tests

pass.

For each of the 16 DRAMs set aside and proven good earlier;  bend up

pin4 a little above the top of the I.C..

Note  that ceramic parts are extremely brittle where a leg joins the

body.  Even with plastic parts try to bend the leg at several places in

a U shape to avoid breakage.

Straighten  the  other  15  pins   on  each DRAM to fit tightly over

another I.C.  beneath it.

Page 1 of 3

Note  that  the  next steps (enclosed in ****** ) must be repeated 4

times, once for each bank.

***********************************************************************

Take  a  row  of  DRAMS out of their sockets and solder another DRAM

piggy back onto each one with pin 4 lifted on the top I.C.   Be careful

to  keep  solder  clear  of  the bottom portion of the lower chip as it

will  be  inserted back into a socket.   The chips must be aligned over

the same number leads; that is pin one over pin one etc..

Note  that  a small tip solder iron must be used and afterward use a

lamp to check the solder joints.

Insert the 8 piggy-back chips into the 8 sockets.

Solder  a  straight  wire  over the top of each row to connect all 8

pin 4's of the piggy back I.C.s.

Solder  a 30 ohm resistor to pin 4 at the end of each piggy back row

on the side nearest to the TMS4500A.

If  you  wish  to check,  the board should still boot up normally to

512k total memory.

***********************************************************************

2.---------------------------------------------------------------------------

Unsolder the 74HC51 and remove it from the board,  if undamaged this

part can be used later.

WHILE THE 74HC51 IS OUT OF THE BOARD:

We will now stack two chips onto the 74HC08 making it 3 high:

Bend  up  pins  8  and  9   on a new 74HC08.   Bend these legs about

perpendicular  to the other legs since we have another chip to stack on

this  one  yet.   Piggy back this on top of the 74HC08 by soldering the

legs  that  are   not   bent.   Prepare  another 74HC08 by lifting legs

2,3,5,6,8,9,10,11,  and  13.   These may be raised a little higher than

on  the  first  chip.   Solder this chip on top of the first piggy back

74HC08.

Remove  pin  6 and pins 8,9,10,11 from a new 14 pin I.C.  socket and

solder  it  in  place  of  the 74HC51 after the piggy back 74HC08s have

been soldered in.

Install an 74HC51 into the new socket.

Page 2 of 3

Wire  HC51-6  to  upper  HC08-10  and  then wire upper HC08-8 to the

plated-thru  hole  at the rear of the 74HC51 that HC51-6 used to go to.

That is the etch near pin 7 of the 74HC51.   That etch is also connected

to pin 4 of the TMS4500A.

3.---------------------------------------------------------------------------

Wire HC368-2 to HC51-9,10,and 11 (all 3 pins tied together) {Add 19}

Wire HC51-8 to upper HC08-9

If  you  wish  to check,  the board should still boot up normally to

512k total memory.

Prepare   a   new   74HC138   by   carefully   bending    up    pins

4,7,9,10,11,12,13,14,  and  15.   You  may remove pins 7,9,10,and 11 if

you  wish.  Piggy back this chip atop the existing 74HC138 by soldering

the  leads  which were not bent or removed.

Wire piggy back HC138-4 to HC51-5. {-add 19}

4.---------------------------------------------------------------------------

Wire  upper  HC138-15  to upper HC08-2 and upper HC08-3 to the first

piggy back 30 ohm resistor.

Wire  upper  HC138-14 to upper HC08-5 and upper HC08-6 to the second

piggy back 30 ohm resistor.

Wire  upper HC138-13 to #middle# HC08-9 and #middle# HC08-8 to the third

piggy back 30 ohm resistor.

Wire  upper  HC138-12  to   upper   HC08-13 and upper HC08-11 to the

fourth  piggy  back 30 ohm resistor.   The PCjr should now post to 640k

total memory and should have 736K usable.

Run memory diagnostics for the full 640k and verify that they pass.

5.---------------------------------------------------------------------------

On  newer  Racore  units the extender board that connects its memory

board  to  the  sidecar has a solid ground plane etched over the bottom

surface.   My  system  didn't   and   as  a result there was electrical

interference  with  the lower floppy disk drive,  even before upgrading

memory.   The fix is to install a thin metal shield under your extender

board and solder a wire from it to a ground pin on the extender.

Page 3 of 3