Copyright (c) Matrox Electronic Systems, 1990-1995.

Image-series User's Kit

-----------------------

Supports Image-1280, Image-640 or Image-LC imaging board, ISA or EISA bus.

Release 3.51c September 1995.

This file describes the contents of the release diskette and their usage.

The Image-Series User's Kit includes some utility programs. It includes:

README.DOC   -This file.

SHELL.OUT    -The Image-Series resident software (Image-Shell).

FPGA.OUT     -Control logic program to be used with XILOAD.exe

*.EXE        -Image-Series utilities for DOS/Windows 3.1, described below.

UTILNT\*.EXE -Image-Series utilities for Windows NT, described below.

INTER.INI    -INTER initialization commands and all of the Image-Series

predefined constants. This file is read at start-up time.

INTER.HLP    -Text file for the help command.

INTER.DOC    -Contains INTER documentation.

.\CAMERA     -Contains some sets of logical camera parameters and

a .TXT file that describes the characteristics of

each logical camera.

.\MONITOR    -Contains .VAL files for the SETVID utility.

.\IMCFG      -Contains .CFG configuration files for an EISA setup.

.\DIAG       -Contains a diagnostic program and a help file (READ.ME)

for IMDIAG.EXE.

.\VGAALIGN   -Contains VGAALIGN utility.

.\TUTOR      -Tutorial and macro examples.

INTER: (inter.exe)

------------------

The INTER utility is an Image-Series command interpreter for DOS. It allows

you to access all the Image-Series commands and to build them into

macros that can then later be used. It has advanced interactive features

to simplify command entry. For more details, refer to Chapter 2 of the

Image-Series Utilities Manual.

IMDOWNL: (imdownl.exe)

----------------------

The IMDOWNL utility downloads the Image-Shell to the Image-Series

Local RAM. For more details, refer to Chapter 4 of the Image-Series

Utilities Manual.

SETVID: (setvid.exe)

--------------------

The SETVID program allows you to modify the video parameters of

the Image-Shell so that it will work with different monitors and

display resolutions. Configuration data is contained in files with a

.VAL extension in the \MONITOR subdirectory. For more details, refer

to Chapter 4 of the Utility Manual.

SET_CAM: (set_cam.exe)

----------------------

The SET_CAM utility allows the user to replace any existing logical camera

included in the shell with one of the sets of logical camera parameters

made available in the \CAMERA subdirectory (.CAM files). In the \CAMERA

subdirectory, there is a .TXT file that describes the characteristics of each

set of parameters.

IMLDCAM: (imldcam.exe)

----------------------

The IMLDCAM utility temporarily replaces a logical camera in the currently

running shell with one of the sets of logical camera parameters made

available in the \CAMERA subdirectory (.CAM files).

IMLDVID: (imldvid.exe)

----------------------

The IMLDVID utility temporarily replaces the video parameters in the

currently running shell with one of the sets of video parameters made

available in the \MONITOR subdirectory (.VAL files).

XILOAD (xiload.exe)

----------------------

The XILOAD utility has been implemented to permit the download of the

FPGA.out file in the programmable control logic of the Image-LC.

EISA configuration files: (*.CFG)

---------------------------------

These files are used by your EISA configuration utility to install

the IM-1280/EISA or IM-640/EISA board set. Please consult the documentation

for your EISA system configuration utility.

PIXDUMP: (pixdump.exe)

----------------------

The PIXDUMP utility for DOS copies a specified section of the frame buffers

to the Host screen. This utility allows you to view the actual pixel

values in the frame buffers. This utility can be useful during debugging.

For more details, refer to Chapter 3 of the Image-Series Utilities Manual.

IMLOAD: (imload.exe)

--------------------

The IMLOAD utility copies an image from a disk file to the frame

buffers. For more details, refer to Chapter 4 of the Image-Series Utilities

Manual.

IMDUMP: (imdump.exe)

--------------------

The IMDUMP utility copies an image from frame buffers to a disk file.

For more details, refer to Chapter 4 of the Image-Series Utilities Manual.

SENDIM: (sendim.exe)

--------------------

The SENDIM utility sends a file with command opcodes to the Image-Series

Write FIFO.  This utility is also useful for downloading character fonts

and other large data sets. For more details, refer to Chapter 4 of the

Image-Series Utilities Manual.

KEY (key.exe)

----------------------

The KEY utility selects the VGA keying mode on the Image-LC.

IMDIAG: (.\DIAG\imdiag.exe)

-------------------------

The IMDIAG utility allows you to run a general diagnostic on an Image-Series

board set.

VGAALIGN (.\VGAALIGN\vgaalign.exe)

----------------------------------

The VGAALIGN utility is used with the Image-LC to fine tune the display

parameters.