S3 DOS Applications, OS/2 and
SCO Drivers Documentation

This document contains installation instructions for the following types of executable S3Ô drivers:

DOS Applications

OS/2Ò 2.0

SCOÒ OpenDesktopÒ

Instructions are provided for building DOS applications drivers from source code. Note that source code must be purchased separately from the executable drivers and may not be included in this package. The descriptions for some drivers contain instructions for their optimal use. In addition, a description is provided for the OS/2 MINI PORT application, which allows some driver customization.

In general, the instructions apply equally to the S3 86C801, 86C805, 86C928 and chips belonging to the same families, e.g., PS86C805 and 86C928PCI. Where an instruction applies to a particular chip or chips, this is noted.

Ó 1992, 1993 S3 Incorporated. All rights reserved. No part of this publication may be reproduced, stored in a retrieval system, or transmitted in any form or by any means, electronic, mechanical, photocopying, or otherwise, without the prior written consent of S3 Incorporated, 2770 San Tomas Expwy., Santa Clara, CA 95051-0968. S3, the S3 Corporate Logo and True Acceleration are trademarks of S3 Incorporated. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Incorporated reserves the right to make changes in the product design without reservation and without notice to its users.

# Table of Contents

µS3 DOS Applications Drivers	4

CADKEY 5.0/DATACAD 4.06 DRIVER	5

Cadkey Driver Installation	5

To Uninstall Cadkey	6

DataCAD Driver Installation	6

To Uninstall DataCAD	7

Building the CADKEY/DataCAD Drivers	7

CADVANCE 3.51 DRIVER	8

Cadvance Driver Installation	8

Building the CADVANCE Driver	8

GENERIC CADD 6.0 DRIVER	10

Generic CADD Driver Installation	10

Building the Generic CADD Driver	10

AUTODESK ADI 4.2	12

Features	12

Aquila Display List and Special Features for AutoCAD	19

Microstation PC Driver (Protected Mode 4.0)	24

MicroStation PC Driver Installation	24

Building the Protected Mode MicroStation PC Driver	25

Microstation PC Driver (Real Mode 3.3)	26

MicroStation PC Driver Installation	26

Building the Real Mode MicroStation PC Driver	27

VersaCAD 7.0 Driver	28

Driver Installation for VersaCAD 7.0 Real mode:	29

Driver Installation for VersaCAD 386 7.0:	29

Building the VersaCAD Driver	29

Lotus 1-2-3 2.3 Driver	31

Lotus 1-2-3 2.3 Driver Installation	31

Building the Lotus 1-2-3 2.3 Driver	31

Microsoft Word 5.0 Driver	33

Microsoft Word 5.0 Driver Installation	33

Building the Microsoft Word 5.0 Driver	33

WordPerfect 5.1 Driver	34

Word Perfect 5.1 Driver Installation	34

Building the WordPerfect 5.1 Driver	34

S3 OS/2 Drivers	35

Installation Considerations	35

Installation Instructions	35

OS/2 2.0 MINI PORT OEM Guide	37

Introduction	37

Mode Tables	38

Customization Example	46

Building Mechanics	47

DSPINSTL Scripts	48

S3 SCO Open Desktop Drivers	54

Preface	54

Contents	54

Software Requirements	54

Installation	55

Installation Procedure	55

Release Notes	56

Supported Display Modes	56

Problem Fixed in Version 3.02	56

Known Limitations	56

Creating Grafinfo Files	57

Introduction	57

Grafinfo Parameters Supported	57

Parameter Descriptions	58

Grafinfo Procedures Supported	60

Sample Grafinfo File	60

# S3 DOS Applications Drivers

This section contains the installation instructions and, where applicable, usage and performance tips for the following drivers:

·	CADKEYÒ v5.0/DataCadÒ v4.06

·	CadvanceÒ v3.51

·	GenericÒ CADDÒ v6.0

·	AutodeskÒ ADIÔ 4.2 Protected mode:

- AutoCADÒ 11/12

- 3D StudioÔ 1.0/2.0

- AutoShadeÒ 2.1

·	MicrostationÔ PC:

- Protected Mode v4.0

- Real Mode v3.3

·	VersaCADÔ Designer 7.0 & VersaCAD 386 7.0

·	LotusÒ 1-2-3Ò 2.3

·	MicrosoftÒ Word 5.0

·	WordPerfectÒ 5.1

## CADKEY 5.0/DATACAD 4.06 DRIVER

The following resolutions are supported:

| Resolution | Colors | Video Memory (MBytes) |
| --- | --- | --- |
| 640 x 480 | 256 | 0.5 |
| 800 x 600 | 16 | 0.5 |
| 800 x 600 | 256 | 1 |
| 1024 x 768 | 16 | 0.5 |
| 1024 x 768 | 256 | 1 |
| 1280 x 1024 | 16 | 1 |

This driver is a TSR (Terminate and Stay Resident) program that must be invoked BEFORE Cadkey or DataCAD will run. The driver will accept certain command line options that will change the behavior of the driver. These options will allow you to remove a previous copy of the driver from memory, or to install the driver at a different interrupt vector.

To remove a previous copy of the driver, use the '-r' option.

Example

CK_911 -r	Removes the driver (if installed) from system memory.

This driver will install itself on interrupt 60h by default. To install the driver at a different interrupt vector, use the '-f##' option, where '##' is replaced by a number between 60 and 67.

Examples

CK_911	Installs the driver at interrupt 60h.
	CK_911 -f62	Installs the driver at interrupt 62h.

### Cadkey Driver Installation

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2. 	Copy the Cadkey driver from the diskette into the sub-directory which contains Cadkey. For example, if Cadkey is located on drive C: in the \CADKEY sub-directory, type the following:

COPY A:\CADKEY\CK_911.EXE C:\CADKEY

3.	Change to the Cadkey subdirectory on the hard-disk and create a backup of the GRDEV.DAT file by renaming GRDEV.DAT to GRDEV.BAK.

4.	Create a new GRDEV.DAT by appending the GRDEV.DAT file on the diskette to the GRDEV.BAK file. To do this, type the following:

COPY GRDEV.BAK+A:\CADKEY\GRDEV.DAT GRDEV.DAT

5.	Run the Cadkey CONFIG program and make the following selections:

a.	Select "Set graphics options".

b.	Select "Select graphics device" option.

c.	Select "S3 86C911 (86C911)".

d.	Select the appropriate video resolution for the monitor installed.

e.	Select the rest of the graphics options, depending upon user preferences.

f.	Exit, saving the new configuration.

### To Uninstall Cadkey

1.	Change to the \CADKEY sub-directory.

2.	Enter the following command:

COPY GRDEV.BAK GRDEV.DAT

3.	Delete the file CK_911.EXE.

4.	Run the Config program and select a different video driver.

### DataCAD Driver Installation

1.	Insert the S3 DOS Driver diskette in the appropriate diskette drive.

Without EMS Memory

2.	Copy the Cadkey driver from the diskette into the sub-directory that contains the DataCAD drivers. For example, if DataCAD is located in the MTEC\DRV subdirectory on drive C: and the diskette is in drive A:, type the following:

COPY A:\CADKEY\CK_911.EXE C:\MTEC\DRV

With EMS Memory

2.	Copy the Cadkey driver from the diskette into the sub-directory that contains the DataCAD drivers. For example, if DataCAD is located in the MTEC\DRV subdirectory on drive C: and the diskette is in drive A:, type the following:

COPY A:\CADKEY\DL_911.EXE C:\MTEC\DRV

4.	Change to the MTEC\DRV sub-directory and rename the driver files DRIVERS.GD2 and DRIVERS.GD4 to DRIVERS2.BAK and DRIVERS4.BAK respectively.

5.	Create a new DRIVERS.GD2 and DRIVERS.GD4 by appending the DRIVERS.GD2 and DRIVERS.GD4 files on the diskette to the DRIVERS2.BAK and DRIVERS4.BAK. To do this type in the following command lines:

COPY DRIVERS2.BAK+A:\DRIVERS.GD2 DRIVERS.GD2
COPY DRIVERS4.BAK+A:\DRIVERS.GD4 DRIVERS.GD4

6.	Change to the \MTEC sub-directory on the hard disk.

7.	Run the DataCAD CONFIG program, selecting the "S3 86C911 (86C911)" as the display driver.

a.	Select the appropriate video resolution for the monitor installed.

b.	Select the rest of the graphics options, depending upon user preferences.

c.	Exit, saving the new configuration.

### To Uninstall DataCAD

1.	Change to the \MTEC\DRV directory.

2.	Enter the following commands:

COPY DRIVERS2.BAK DRIVERS.GD2
COPY DRIVERS4.BAK DRIVERS.GD4

3.	Delete the file CK_911.EXE or DL_911.EXE.

4.	Change to the \MTEC directory.

5.	Run the CONFIG program and select a different video driver.

### Building the CADKEY/DataCAD Drivers

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\cadkey c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## CADVANCE 3.51 DRIVER

This driver is loaded into memory by Cadvance and requires no intervention on the part of the user. Cadvance will also remove the driver from memory upon completion, thus conserving DOS memory.

This driver will work as a rendering device if sufficient memory is installed and the driver is using a 256 color mode. The resolution will be the same as the resolution used in the 2D and 3D modes.

The driver will default to the highest resolution possible. If this is not what you desire, the resolution may be overridden by setting an environment variable as follows:

SET S31_911=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors (911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

### Cadvance Driver Installation

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2. 	Copy the Cadvance driver from the diskette into the sub-directory which contains Cadvance. For example, if Cadvance is located on drive C: in the \CAD sub-directory and the diskette is in drive A:, type the following:

COPY A:\CADVANCE\CDV911.DRV C:\CAD\GS.DRV

### Building the CADVANCE Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\cadvance c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## GENERIC CADD 6.0 DRIVER

The driver is loaded and unloaded in memory by CADD and has no TSR (Terminate and Stay Resident) considerations. If there is no need for the screen save function in Generic CADD (driver LGEN911.VGD), the smaller driver (SGEN911.VGD) is recommended. This will free up an additional 11 KBytes of drawing space if no EMS is installed, or around 88 KBytes if there is EMS memory.

The driver will default to the highest resolution possible. If this is not what you desire, the resolution may be overridden by setting an environment variable as follows:

SET CADD_INIT=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors (911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

### Generic CADD Driver Installation

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2.	Copy the driver you wish to use from the \GENCAD sub-directory on the driver disk to the sub-directory which contains the application. For example, type

COPY A:\GENCAD\LGEN911.VGD C:\LEVEL2

to copy the driver with screen save function for use with CADD level 2.

3.	After the desired driver is copied into the appropriate sub-directory, run CONFIG, or 3DCONFIG if 3D Drafting is used, to configure the application. Configure the Generic product for "S3 86C911" as the display driver. If CADD 6.0 is used, first select the "Other VGD's" option, then select "S3 86C911."

### Building the Generic CADD Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\gencad c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## AUTODESK ADI 4.2

The AquilaÔ driver is a professional quality PADI (Protected mode Autodesk Driver Interface) combined display and rendering device driver to be used with AutoCAD Release 11/12, 3D Studio 1.0/2.0, and AutoShade 2.1.

### Features

·	Protected mode operation, PADI 4.2 specification

·	32 bit display list for "no regen" operation

·	Display list operation with 16 active viewports, including paperspace viewports

·	Birds-Eye and Edge-Pan features for convenient zooms and pans

·	Partial redraws on overlapping viewports

·	Redraw list for maximum redraw speed with Release 11

·	Intelligent clean-up (Erase Bufferä ) to eliminate "flashing"

·	Display list memory meter

·	AutoCAD memory usage statistics

·	Paging-Alarmä - disables display list usage when paging become excessive

·	Reliable dual screen operation

·	Variable font support- allows fonts up to 16 by 30 in size

·	International font support

·	User configurable screen colors

·	15 and 16 bit "Band-Freeä " Rendering support

·	24 and 32 bit Rendering support

·	Text screen emulation window for Release 12

The following display and/or rendering resolutions are supported:

| Resolution | Colors | Minimum Video Memory (MBytes) | Display = D Rendering =R | Notes |
| --- | --- | --- | --- | --- |
| 640 x 480 | 256 | 0.5 | D,R |  |
| 640 x 480 | 32K | 1 | R |  |
| 640 x 480 | 64K | 1 | R |  |
| 640 x 480 | 16.7M | 1 | R | Not available on 911 boards or 928 A-D rev boards with Brooktree RAMDAC |
| 800 x 600 | 16 | 0.5 | D |  |
| 800 x 600 | 256 | 1 | D,R |  |
| 800 x 600 | 64K | 1 | R | Not available on 911,924 or 928 A-D rev boards |
| 800 x 600 | 16.7M | 2 | R | Not available on 911,924 or 80x or 928 A-D rev boards |
| 1024 x 768 | 16 | 0.5 | D |  |
| 1024 x 768 | 256 | 1 | D,R |  |
| 1024 x 768 | 64K | 2 | R | Not available on 911,924 boards |
| 1024 x 768 | 16.7M | 4 | R | Not available on 911,924 or 80x or 928 A-D rev boards |
| 1152 x 864 | 256 | 1 | D,R | Not available on 911,924 boards or on 80x A,B rev or 928 A-D rev |
| 1280 x 960 | 16 | 1 | D | 911, 924 boards only |
| 1280 x 1024 | 16 | 1 | D |  |
| 1280 x 1024 | 256 | 2 | D,R |  |
| 1280 x 1024 | 64K | 4 | R | Not available on 911,924 or 80x or 928 A-D rev boards |

Installation

1.	Create a directory on your hard drive for the Aquila files by entering

md \aquila

Move to that directory by entering

cd \aquila

2.	Copy all files from the \ADI42 subdirectory on the S3 DOS Drivers diskette to this subdirectory, e.g.,

copy a:\ADI42\*.*

Basic Configuration

1.	Add the following 3 lines to your AUTOEXEC.BAT:

Set dspadi=c:\aquila\rcp_s3.exp
Set rcpadi=%DSPADI%
set rdpadi=%DSPADI%

This will cover all Autodesk applications except AutoCAD Release 12.

2.	Refer to your application's Installation and Performance Guide for information about other environment variables, configuring other non-display device drivers and system configurations that are necessary to optimize the performance of your application.

3.	Aquila will detect the Autodesk application that it is being configured for and will provide correct display driver defaults for your applications. By accepting the default parameters, your driver will be configured correctly. This will work with AutoCAD, AutoShade, or 3D Studio.

Font Configuration for AutoCAD and AutoShade

1.	This driver will use the 8-bit international font in the VGA BIOS (8x16) or, if present, an Aquila optional font file. Font files are available in sizes up to 16 x 30. The README file lists available font files and a brief description of each.

2.	The driver searches for the file AQUILA.FNT in the current directory. If not found, it searches for the file specified in the DOS environment variable AQUILA_FONT. For example,

set aquila_font=c:\aquila\9x15.fnt

If the font file is readable, it will be used; otherwise Aquila will borrow font data from the VGA BIOS.

3.	Please note that font files can only be used with AutoCAD and AutoShade, while 3D Studio will only use the internal 8x16 VGA font. If Aquila font files are used with 3D Studio, characters such as arrowheads may look strange or different.  If you are using AutoCAD and 3DStudio on the same system, you may want to disable an active Aquila font before entering 3DStudio by entering

SET AQUILA_FONT=

AutoCAD Release 11 Configuration

1.	Set the environment variables as described above under Basic Configuration.

2.	Set the font variable as described above under Font Configuration. 	Rerun your batch files if needed to initialize the variables.

3.	Start AutoCAD by moving to the \ACAD directory and typing ACAD.

4.	If AutoCAD has not yet been configured or your acad.cfg has been deleted, skip to 6, below.  Otherwise, select option #5 "Configure AutoCAD".

5.	Select option #3 "Configure video display".

6.	Select option #1 "ADI P386 v4.0/4.1 display" which will display the Aquila configuration menu.

7.	Choose 1 to select a display resolution mode. If your system is capable of dual monitor operation, you will be asked if you wish to use this option.

Option 1 is the only display driver option that must be set before using AutoCAD 11.  For options 2 through 9, you may accept the pre-set defaults by choosing 0 to exit the Aquila configuration menu, or adjust them as follows for a customized display configuration.  Details regarding these custom Aquila display list options are provided later in this document.

8.	Choose 2 to activate the color menu to customize colors used for the AutoCAD graphics screen.

9.	Choose 3 to disable the Graphics Flip-screen.  This feature is available for Release 12 only.

10.	Choose 4 to disable or adjust the Display List memory management options.

11.	Choose 5 to disable or adjust the Erase-buffer size (default is enabled for "flash-less" redrawing during editing).

12.	Choose 6 to enable or adjust the sensitivity setting for the Page-Alarm (default is off).

13.	Choose 7 to adjust or enable the Edge-pan option (default is off).

14.	Choose 8 to disable or change the size of the Birds-eye (default is 25% of the total screen size).

15.	Choose 9 to configure the digitizer or mouse button to use for entering commands. The default is the transparent Birds-eye command "'AQBIRD", as the recommended double-click command. This will allow you to bring up the Birds-eye anywhere on the graphics screen while in the middle of an AutoCAD command.

16.	Exit the Video Display Configuration menu.

17.	Configure pointer, printer and plotter as needed.

18.	Return to the main AutoCAD menu, saving the configuration results

AutoCAD Release 12 Configuration

AutoCAD Release 12 uses a much different method of locating device drivers than Release 11. Release 12 will search directories for device files, ignoring the DSPADI= environment variable. If you are installing or configuring Release 12 for the first time, please pay careful attention to the following setup instructions.

1.	Set the environment variable in both the ACADR12.BAT and AUTOEXEC.BAT files as follows:

set ACADDRV=C:\AQUILA;C:\ACAD\DRV

2.	Set the font variable as described above under Font Configuration. Rerun your batch files if needed to initialize the environment variables.

3.	Start AutoCAD by moving to the \ACAD directory. Enter ACAD if you are configuring for the first time or ACAD -R if you are reconfiguring and select item #3 to activate the Video Display driver menu.

4.	 From the available display drivers, select "S3 ADI v4.2 Display and Rendering by ATC GraphicsÔ".

5.	Choose 1 to display the list of available resolutions.  Select the AutoCAD S3 Display resolution mode you want. If your system is capable of dual monitor operation, you will next be asked if you wish to use this option.

Option 1 is the only option that must be selected before using AutoCAD 12.  For options 2 through 9, you may accept the pre-set defaults by choosing 0 to exit the Aquila configuration menu, or adjust them for a customized display configuration.  Details regarding the special Aquila display list features are provided later in this document.

6.	Choose 2 to start the color menu to customize colors used for the AutoCAD graphics screen.

7.	Choose 3 to enable or disable the "Flip-screen" feature (default is on for Release 12).  Flip-screen is automatically disabled if you selected dual monitor operation.

8.	Choose 4 to disable or adjust the Display List memory management options.

9.	Choose 5 to disable the Erase-buffer size (default is enabled for "flash-less" redrawing during editing).

10.	Choose 6 to enable or adjust the sensitivity setting for the Page-Alarm (default is off).

11.	Choose 7 to enable or adjust the Edge-Pan feature. (Default is off.)

12.	Choose 8 to disable or change the size of the Birds-Eye (default is 25% of the total screen size).

13.	Choose 9 to configure the digitizer or mouse button to use for double-click entering of your selected command. The default  is the recommended transparent Birds-eye command "'AQBIRD", as this will allow you to bring up the Birds-eye anywhere on the graphics screen while in the middle of an AutoCAD command.

14.	Exit the Aquila Display configuration menu and the Video Display configuration menu.

15.	Configure pointer (mouse or tablet), printer and plotter as needed.

16. 	Return to the main menu and save the configuration results.

AutoCAD 12 AVE Configuration

The Aquila driver is a combined display and rendering device driver and supports 15, 16, 24 and 32 bit rendering within AutoCAD.  AutoCAD's AVE Rendering must be configured for the Aquila driver.

1.	If you are reconfiguring AutoCAD, delete your current AVE.CFG file.

2.	Once in AutoCAD, either type RCONFIG at the AutoCAD Command prompt, or select from the pulldown menus, RENDER/PREFERENCES/Reconfigure.

3.	When you configure the rendering  driver for AutoCAD Render, select option 1, AutoCAD's configured P386 combined display/rendering driver. (Do not select option 2).

4.	Select the S3 Rendering resolution you want from the menu list of available resolutions supported by your system.  Your selection must be compatible with any limitations related to your current AutoCAD display configuration.

·	All resolutions listed will work when rendering to a full-screen with a single monitor system.

·	With a dual-monitor configuration, select rendering to a viewport (because currently AutoCAD 12c2 does not return properly from rendering to a separate screen).

·	If your AutoCAD display resolution is set to any 16-color resolution, then you must render to the full screen instead of a viewport.  Any resolution may be selected.

·	If your AutoCAD display resolution is set to any 256-color resolution and you plan to render to a 256-color viewport, select the same resolution for your display and renderings.   When rendering to a viewport, AutoCAD's display driver controls the rendering resolution.

5.	Select whether you wish to render to a viewport or a full screen (see 4 above for limitations).

6.	Configure any hardcopy device as needed.

7.  	Exit the configuration menu and save your changes.

AutoShade 2.1 Configuration

1.	Install the Aquila files as described under installation.  Set the environment and font variables as described above under Basic and Font Configuration. Rerun your batch file if needed to initialize your environment  variables.

2.	Start AutoShade 2.1, with the reconfiguration command SHADE /R.

3.	Select the correct pointing device.

4.	Select option #2 "P386 Autodesk Device Interface display driver".

5.	Choose option #1 to activate the list of display resolution modes for the interactive screen.  Select one.

6.	Choose option #2 to activate the Color menu and select colors to use for Menus, Pop-ups, Background, etc.

7.	Save and exit the colors menu.

8.	Select option #2 "P386 Autodesk Device Interface rendering driver".

9.	Choose a resolution mode for the rendering (shading and RenderMan) screen.

10.	If you select the 32K or 64K color modes, you will be asked if you wish to enable "Band-Free" mode which allows the best color quality possible in a 15 or 16 bit device. Select Low, Medium or High.

11.	Answer the next 3 questions with a "YES", since only one screen is used, and must be redrawn upon switching.

12.	Select a hard copy device option.

13.	Configuration is complete and the interactive screen is displayed.

3D Studio 1.0 Configuration

Verify that the environment variables listed under Basic Configuration, above, are initialized before proceeding with your 3D Studio configuration.

1.	Go to your 3D Studio 1.0 subdirectory, e.g.,\3DS, and edit the 3DS.SET file.

2.	Change the line that starts with DEFAULT-DISPLAY = "VGA320X200" to read DEFAULT-DISPLAY = "RCPADI".

3.	Change the line that begins with MAIN-DISPLAY = to read MAIN-DISPLAY = RCPADI. Delete any leading semicolon (;) that may be present.

4.	Change the line that begins with MATERIAL-DISPLAY = VGA320X200 to read MATERIAL-DISPLAY = RCPADI.

5.	Save the file and exit the editor.

6.	Delete the file 3DADI.CFG, if present.

7.	Start 3D Studio with the command 3DS.

8.	Select a 256-color resolution for the Main display. This is the mode that will be used for the 2D Shaper, 3D Lofter, 3D Editor, and Keyframer.

9.	Select a 256-color resolution for the Materials Editor. Use of a resolution greater than 640 x 480 will not increase the usefulness of the Materials Editor at all.

10.	You should now find yourself inside the 3D Editor of 3D Studio.

11.	The first time that you try to render a drawing or view an image, you will be asked to select a resolution for Rendering. If you select the 32K or 64K color modes, you will be asked if you wish to enable "Band-Free" mode. This allows the best color quality possible in a 15- or 16-bit device. Select Low, Medium or High. Configuration is now complete.

3D Studio 2.0 Configuration

Verify that the environment variables listed under Basic Configuration, above, are initialized before proceeding with your 3D Studio configuration.

1.	Go to your 3D Studio 2.0 subdirectory, e.g., the \3DS directory, and edit the file 3DS.SET file.

2.	Change the line that starts with DEFAULT-DISPLAY = VGA320X200 to read DEFAULT-DISPLAY = RCPADI.

3.	Change the line that begins with MAIN-DISPLAY = to read MAIN-DISPLAY = RCPADI. Delete any leading semicolon (;) that may be present.

4.	Change the line that begins with MATERIAL-DISPLAY = VGA320X200 to read MATERIAL-DISPLAY = RCPADI.

5.	Save the file and exit the editor.

6.	Delete the file 3DADI.CFG, if present.

7.	Start 3D Studio with the command 3DS. Answer Y to the question about having a VGA or SVGA.

8.	Select a 256-color resolution mode for the "ADI main display". This is the mode that will be used for the 2D Shaper, 3D Lofter, 3D Editor, and Keyframer.

9.	Select a 256-color resolution for the "ADI MEDIT", Materials Editor. Use of a resolution greater than 640 x 480 will not increase the usefulness of the Materials Editor at all.

10.	Answer N to the question about "Will RDPADI render to a separate screen?".

11.	Select a resolution mode for the Rendering "RDPADI" Device. If you select the 32K or 64K color modes, you will be asked if you wish to enable "Band-Free" mode. This allows the best color quality possible in a 15 or 16 bit device. Select Low, Medium or High. (This rendering mode is not activated in a single monitor system.)

12.	Answer the question about "Will RCPADI render to a separate screen?". If using Aquila for an 8514/A, the answer is "yes", for all others, the answer is "no".

13.	Select a resolution mode for the Rendering "RCPADI" Device. If you select the 32K or 64K color modes, you will be asked if you wish to enable "Band-Free" mode. This allows the best color quality possible in a 15 or 16 bit device. Select Low, Medium or High.

14.	Configuration is now complete.

### Aquila Display List and Special Features for AutoCAD

AutoCAD Screen Layout

·	Current Color Display - The rectangle in the top left of the screen displays the current drawing color.

·	Logo - The manufacturer's logo is displayed near the lower right corner of the screen.

·	Display Status Area - This area is located below the Logo Area and is present only when the display list has been enabled.  It contains three lines of text as follows:

System Memory Status - The topmost line of text in the Display Status Area  provides a percentage of total system memory that is being used by AutoCAD. AutoCAD may start to slow down when this amount exceeds 100%. After this figure exceeds 150%, "paging" will occur.

Display/Screen Lists Memory Status - The middle line of text indicates memory usage by the Aquila display list and screen list.

numberK		indicates amount of memory currently used by display list

Empty		indicatesthere is no drawing in memory, anew drawing, this line

Overflow 	indicates the memory required by the display list exceeds available display list memory

Disabled		indicates that the display list has been disabled because excessive paging has occurred

Display List Status - The third or lowest line of the Display Status Area lists abbreviations to indicate the current status of display list features.  Key letters are present only when the display list is enabled.

D	32 bit display list data is in use.

d	16 bit display list data has been selected.

Z	Zooming from the display list is allowed.

z	Pspace and perspective zooms by AutoCAD. Birds-eye will not work in viewport.

S	Redraw list is enabled.

s	Redraw list is turned off.

P	Page alarm enabled and paging occurred recently.

p	Page alarm enabled and no paging has occurred.

E	Edge-pan enabled. Edge pan is impending.

e	Edge-pan enabled but none impending.

Display List Memory Management Configuration for AutoCAD 11 and 12

This section covers the configuration of the Aquila display list memory management features for AutoCAD available under option #4 of Aquila's AutoCAD display configuration menu. Please pay careful attention to the information presented here, since this will affect the performance of Aquila under AutoCAD.  Some experimentation will be necessary in order to make Aquila work optimally for your particular environment and needs.

1.	While configuring, you must choose a method of memory management for the display list information. Which method you use will depend upon the type and number of drawings you work with during an AutoCAD session.

·	Dynamic - Memory keeps on growing as it is needed. This requires the least amount of operator interaction, and is the most carefree.

·	Static - Memory is a fixed amount that you will specify. When the display list grows bigger than memory limits, the display list will 	be shut off. The static option is a little bit faster, but requires a knowledge of how much memory your drawings will consume.

2.	The first option available in the Display List memory management submenu is "Enable Display List?". Answering "No" will end the display list configuration choices.

3.	If you are configuring for Release 12, you will next be asked "Enable 32 bit regens?"  Enabling this feature will greatly reduce regens,  thereby providing faster operation. The down side of this choice is that Aquila will use more memory for the display list.

4.	Next you will be asked to select a memory management scheme, "Dynamic dlist (automatic)?"

·	Answering "Yes" will select the dynamic scheme. You would then be prompted to select a performance level, "Miserly (1), Normal (2) or High Performance (3)". If you have very little memory in your machine then select Miserly.  If your have lots of memory, select High Performance. All others select Normal.

·	Answering "No" will select the static management scheme.  You would then be asked to provide a memory size to use. If this is the first time it is being configured, Aquila will make a suggestion based upon the amount of memory in your system.

5.	If you are configuring for Release 11, you will be asked if you wish to "Enable redraw list?"  Answering "Yes" allows you to enable a faster redraw method. This will consume some additional memory, but it does improve redraw speeds. If you selected "Static" memory management earlier, then you will be asked to select a memory size for the redraw list. Usually this can be 1/2 to 1/4 of the amount you specified for display list usage.

Display List Feature Configuration for AutoCAD 11 and 12

This section covers the configuration of the Aquila display list features available under options #3 and #5 through #9 of the Aquila display configuration menu for AutoCAD.

Graphics Flip-screen (Option #3)

If enabled, the AutoCAD text screen will appear in the center of the graphics screen.  This eliminates the mode switching delay common to most multi-frequency monitors.  When enabled the graphics Flip-screen will emulate the VGA text window. Use the F1 key to toggle between modes. If you wish to execute any DOS commands, Aquila will switch the screen to a full-screen text mode allowing for full DOS compatibility.

If you are using an after-market TSR product that pops up into the screen during AutoCAD sessions (calculator, plot spooler, etc.) or you wish to use the Print-screen function while in the text screen, then you should not use this option.  Flip-screen is automatically disabled for dual monitor operation.

Erase buffer (Option #5)

Answering Yes to "Enable erase buffer?" brings up a second question "Erase buffer size?". This will determine the number of erased objects it takes before Aquila will start an automatic "cleaning" cycle to remove them from the display list. Select a higher value if you plan on doing much large drawing editing.  The erase buffer can also be disabled, enabled and resized from within AutoCAD by typing "AQEBUF=0"at the command prompt to disable and "AQEBUF=xx" to enable or resize, where xx is the new size.

When AutoCAD erases objects from the drawing database, it erases them from the screen as well. Sometimes this action causes other objects to appear erased as well. The "redraw" command has no effect, but a "regen" causes the objects to appear correctly.  This problem can be solved by using the erase buffer to correct the display corruption that occurs when objects are erased from the drawing. Normally, AutoCAD will request that the driver erase a line from the screen by drawing it in black. This causes the erased object to flash on and off the screen during redraws.

Another side effect is that objects near the erased one may appear to be incomplete. To correct this in the fastest manner possible, Aquila will keep a list of erased vectors, and erase them only when the buffer fills. This speeds up block erasures considerably.  The custom command, AQCLN, discussed below, can also be used to clear the erase buffer.

There are certain after market software packages for AutoCAD, such as CAD Overlay, which rely upon the ability to draw objects in black (background color). If you are using one of these packages and the screen images appear distorted, then you need to disable the erase buffer.

Paging Alarm (Option #6)

Answering "Yes" to "Enable Page-Alarm?" will bring up a second prompt about the sensitivity setting for the Page-Alarm.

When AutoCAD's memory requirements exceed the amount of physical memory available in your machine, it begins "paging" in order to gain extra memory. Paging means that certain memory areas are traded back and forth with a "swap file" on the hard drive. Due to the difference in access speed between main memory, and this "virtual" memory that is on the hard drive, AutoCAD begins to slow down, often to the point of being unusable. To combat this, memory is used very sparingly, and only when absolutely necessary. As a second line of defense against paging, when the amount of memory paged out to disk exceeds several preset thresholds, Aquila begins shutting down extra functions, and returns the memory to AutoCAD.

When paging activity exceeds the first threshold, the screen-list feature is disabled and all associated memory is returned to AutoCAD. At this point, the screen list status line, the lowest line,  will disappear from the status window. Should paging activity continue, Aquila will disable the display list feature and return all of that memory to AutoCAD. When that has been done, the middle line of text in the status area will say "Disabled", to indicate that all display list functionality has been terminated.The driver will continue to function as normal, but redraws will be slower. However, AutoCAD itself will now be much faster than it was when the paging was occurring. This allows you to complete your drawing session in much less time.

Edge Pan (Option #7)

Edge-pan is used to move around while zoomed into a drawing. By leaving the cursor near the border of the currently active viewport, Aquila will automatically begin to pan in that direction. This only begins if the cursor is left idle for a predetermined length of time. The default is Edge-pan disabled.

Experimentation with the Edge-pan settings will yield the best results for your needs.  There are nine custom Edge-pan commands provided with the Aquila driver.  Edge-pan settings may be adjusted either in the configuration menu or within AutoCAD by typing at the command prompt "AQEPAN ON" or " AQEPAN OFF".  Please refer to the list of custom Aquila commands given below for additional custom Edge Pan commands.

Birds-eye View (Option #8 & 9)

The Birds-eye feature allows fast transparent zooms and pans. The Birds-eye can be activated either by typing "'AQBIRD" or by double-clicking on the mouse or digitizer button specified during configuration.

When activated, the Birds-eye window will appear under the current cursor location. The window will show a small overall view of the drawing that AutoCAD has currently placed in Aquila's display list. Within the Birds-eye window a highlighted rectangle indicates the currently viewed area, and an outline rectangle indicates the area that will be shown if a Birds-eye zoom is completed. If this rectangle contains an "X" in the middle, then the Birds-eye is in pan mode. If this rectangle contains an arrow, then the Birds-eye is in zoom mode.

Pressing the pick button on your mouse or digitizer (usually the left button for a mouse) allows you to toggle between zoom and pan modes in the Birds-eye window. Pressing the selection button on your digitizer causes the Birds-eye to disappear and the selected view to be used in the currently active viewport. A new zoom will be executed if necessary. Moving the cursor outside the Birds-eye window without making a selection will cancel the function. This Birds-eye will work in multiple viewport drawings, and will appear in the aspect ratio of the currently active viewport.

The Birds-eye can be resized at any time during an AutoCAD session by typing "AQBESZ=xx", where xx is any number between 10 and 50 and represents the percentage of the graphics screen size for the new Birds-eye size.  The image inside the Birds-eye when it is activated can be manipulated without causing a "regen" in Release 12 by using the "AQZWI" command.  By performing a "ZOOM ALL" followed by an "AQZWI" , you will be able to zoom in and out of your drawing as much as possible without causing further regens.

Band-Free Rendering

When using the 15 or 16 bit rendering modes, often times the images appear "banded". This banding or staircasing occurs most often on objects with large sloping or gently curving surfaces. Band-Free rendering improves the image quality in these modes.

The "Band-Free" effect can be controlled as well. It may be set to "Low", "Medium", or "High", depending upon how much of the banding you want to eliminate. This feature will be of limited use to users of AutoShade, but is beneficial to users of Autodesk RenderMan and 3D Studio.

Custom Aquila Commands for AutoCAD

Aquila provides a number of custom commands which provide status information about memory usage for the Display list.  Additional  commands enable the user to reset or redefine Aquila Edge-pan and Birds-eye features from within AutoCAD.  Below is a summary of the custom Aquila commands.

AQHLP	The help screen function displays a list of all the driver local commands, and gives a brief description of each.  This command is best executed from the text or flip-screen.

AQDLQ	Displays on a text-or flip-screen all the viewports currently in use, along with some useful statistics about each one.   This command is best executed from the text or flip-screen.

AQDRQ 	Displays current display resolution and memory allocation statistics (CTRL-L provides the same function). ).  When executed from the command prompt of the graphics screen, only a three-line summary will be given.

AQCLN	Removes objects from display list.  "Cleans" the drawing display.

AQBIRD	Displays Birds-eye window underneath current cursor location.  This command is not designed to work for paperspace or perspective viewports.

AQEPAN ON/OFF	Toggles on and off the Edge-pan feature.

AQPL	Pan to the left by 25 percent.  Regens are never authorized by the pan or zoom in/out Aquila commands.

AQPR	Pan to the right by 25 percent.

AQPU	Pan up by 25 percent.

AQPD	Pan down by 25 percent.

AQZI	Zoom in by 50 percent.

AQZO	Zoom out by 50 percent.

AQZWI	"Zoom way in" while using the same scale factor, will cause a regen. By performing a ZOOM ALL followed by an AQZWI you may zoom around your drawing as much as possible without a regen. (AutoCAD 12 only)

AQBESZ=xx	Resize the Birds-eye window. Valid number is between 10 and 50, and it is a percentage of the graphics screen size.

AQPANSCR=xx	Amount of the viewport scrolled by the edge-pan feature. Valid number must be between 10 and 80, and it is a percentage of the viewport size.

AQPANBRD=xx	Width of the edge-pan border given in number of pixels. Valid number must be between 5 and 50.

AQPANTM1=xx	Amount of initial delay given in number of "clicks" or fractions of a second used before the edge-pan begins. Valid range is 10 to 64.

AQPANTM2=xx	Amount of time before another edge-pan begins. Valid range is 1 to 18.

AQEBUF=xx	Size of the erase buffer. Using 0 turns off the buffer.

## Microstation PC Driver (Protected Mode 4.0)

The first time this driver is used, it will ask the user which video mode to use for MicroStation. It will present the user with a menu of all available resolution modes. One must be selected, and then a configuration file will be created that the driver will use for subsequent MicroStation sessions.

To re-configure the driver, erase the file \USTATION\DRIVERS\MGL911.CFG and run MicroStation again.

To temporarily change the resolution mode used, you may set the environment variable MGL911 as follows:

SET MGL911=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1152 x 864, 256 colors

6	1280 x 1024, 16 colors

7	1280 x 1024, 256 colors

If the driver finds a legal value in the environment, it will use it to override any variable found in the configuration file.

### MicroStation PC Driver Installation

1.	Insert the S3 DOS Driver Diskette in the appropriate diskette drive.

2.	Copy the following files to the \USTATION\DRIVERS directory:

MGL911.EXP

MGL911.MA

MGL911.MGL

3.	Change the current directory to the \USTATION sub-directory, or the one which contains the MicroStation application.

4.	Run the USCONFIG program to select the new device driver.

a.	Select "Display Adapters" entry.

b.	Select "VENDOR SUPPLIED DRIVER" option.

c.	Select "PMGL 86C911 DRIVER" option.

d.	Select second adapter options as needed.

e.	Select "Exit and Save".

f.	Reboot system for installation to take affect.

This driver will work with a Hercules-compatible MDA card as the second graphics device. This can be configured as either the "Primary" (right screen) or "Secondary" (left side) graphics device.

SWAP screen capabilities are supported if a 256 color mode is configured. If the initialization code is unable to determine which mode to use, it will always default to the 640 x 480 x 256 color mode. The driver will exit if it is not able to find an S3-based graphics card.

### Building the Protected Mode MicroStation PC Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

·	MetaWare High C (HC836), version 1.71 or later

·	Phar Lap Linker (386LINK), version 3.0 or later

·	Phar Lap Macro Assembler (386ASM), version 3.0 or later

·	Intergraph Driver Developers Kit for MicroStation PC

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\pmustat c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## Microstation PC Driver (Real Mode 3.3)

The driver is a TSR (Terminate and Stay Resident) driver that must be invoked before MicroStation PC will run. The driver will accept certain command line options that will change the behavior of the driver.

If you have at least 1 MByte of Video RAM,  this driver will support either 2 pages ("swap screen" capability) or will support 256 colors. To select the 256 color option, use the "/noswap" option.

MicroStation 4.0 will run faster (redraws 50% faster) if you have a VCPI memory manager (QEMM or 386MAX) present and correctly configured. To optimize your VCPI memory for use with 4.0, all memory outside the DOS 640K range should be configured as EMS memory.

The driver will default to the highest resolution possible. If this is not what you desire, the resolution may be overridden by setting an environment variable as follows:

SET S31_911=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors (911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

### MicroStation PC Driver Installation

1.	Insert the S3 DOS Drivers Diskette in the appropriate diskette drive.

2.	Copy the following files into the MicroStation Drivers subdirectory on the hard disk from the \USTATION sub-directory on the diskette:

MGLS3.EXE
MGLS3.MGL
REMOVE.EXE

3.	Change to the \USTATION sub-directory.

4.	Run USCONFIG, selecting the "ATC Graphics S3 86C911" driver.

5.	Save the new settings and exit USCONFIG.

6.	Before using MicroStation, execute the TSR driver first:

7.	To reconfigure the driver, remove the previous driver by using the command "Remove". Enter a new MGLS3 in the Drivers directory.

8.	Set the mode number by entering the command "Set S31_911 = <mode>".

9.	Run MicroStation again.

10.	Another way to reconfigure the driver is to reboot the system.

Examples

MGLS3	Installs the driver in 16 color mode. If enough Video RAM is installed, swap screen will be enabled.

MGLS3 /noswap	Installs the driver, telling it to display the maximum number of colors possible.

### Building the Real Mode MicroStation PC Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

·	MetaWare High C (HC836), version 1.71 or later

·	Phar Lap Linker (386LINK), version 3.0 or later

·	Phar Lap Macro Assembler (386ASM), version 3.0 or later

·	Intergraph Driver Developers Kit for MicroStation PC

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\ustation c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## VersaCAD 7.0 Driver

This driver is a TSR type (Terminate and Stay Resident) program. You only need to install it once in memory each time your machine is powered up. For this reason, it is recommended that a call to this driver be placed in your AUTOEXEC.BAT file. The drivers will support VersaCAD Designer 7.0 and VersaCAD 386 7.0. If you are using the real-mode application, the driver used is VC2_911.COM. If you are using VersaCAD 386, the driver used is VC3_911.COM.

The following resolutions are supported:

| Resolution | Colors | Video Memory (MBytes) |
| --- | --- | --- |
| 640 x 480 | 256 | 0.5 |
| 800 x 600 | 16 | 0.5 |
| 800 x 600 | 256 | 1 |
| 1024 x 768 | 16 | 0.5 |
| 1024 x 768 | 256 | 1 |
| 1280 x 1024 | 16 | 1 |

VersaCAD/386 will run faster (redraws 50% faster) if you have a VCPI memory manager (QEMM or 386MAX) present and correctly configured. To optimize your VCPI memory for use with VCAD/386, all memory outside the DOS 640K range should be configured as EMS memory. This means that all the memory that would normally be allocated to "DOS high memory" should be reserved for use as EMS.

EMS Memory Usage

As a default, this driver will reserve 512K of EMS memory when invoked. This will be used to store the display lists needed for fast redraws. While this should be enough memory to handle most drawings, you can tell the driver to reserve more EMS space when the driver is loaded. This is done by setting the environment variable VCAD_EMS= to the number of EMS pages to be reserved. Each EMS page is 16K in size.

Examples

SET VCAD_EMS=64	Causes the driver to reserve 1024K (64 pages * 16K) of memory when the driver is loaded.

SET VCAD_EMS=0	Tells the driver to reserve no pages of EMS for use. This special case tells the driver not to reserve any EMS. In this case, display list processing is disabled.

Display Resolution

This driver will use the resolution selected by the ENVIRO program which ships with VersaCAD. If that mode is unavailable, VersaCAD will not run.

Display List

The main feature of this driver is the built-in display list capability. To take advantage of this feature, VersaCAD must be told that the driver can support display listing, and that you wish to use it. To do this, go into the screen options in ENVIRO, and select the Built-In option under Display List.

To make the most of the display list, use the display list options menu inside VersaCAD by pressing Ctrl-F5.

To minimize the number of "regens", (where VersaCAD sends down each individual line) you should zoom the display all the way in, then back out to where you were in the drawing. After doing this, VCAD will redraw everything from the local display list, (sketches) and zooming and panning (with the arrow keys) will be much faster. This is done by using the "Full" function from the displist list (LIST) menu, then using the "Out" function from the same menu.

To speed up the panning operations (scrolling throughout the drawing can be done with the arrow keys), use a keyboard utility to increase the "Typematic" rate of your keyboard. This will increase the rate at which the scrolling occurs.

### Driver Installation for VersaCAD 7.0 Real mode:

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2.	Change the current drive/directory to the one which contains the VersaCAD application.

3.	Copy the file VC2_911.COM from the \VERSACAD sub-directory on the diskette into the current drive/directory.

4.	Execute VC2_911.COM.

5.	Run the ENVIRO program and select the "S3 86C911" driver. Reboot the system if the installed driver is replacing a previously run driver.

### Driver Installation for VersaCAD 386 7.0:

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2.	Change the current drive/directory to the one which contains the VersaCAD application.

3.	Copy the files VC3_911.COM and VCAD386.CFG from the \VERSACAD sub-directory on the diskette into the current drive/directory.

4.	Execute VC3_911.COM.

5.	Run the ENVIRO program and select the "S3 96C911" screen driver. You will be able to choose between "Single Screen" and "Dual Screen" configurations, along with the various resolutions available. Reboot the system if the installed driver is replacing a previously run driver.

### Building the VersaCAD Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\versacad c:\driver\source

The source code can now be modified as desired. To build the executable code from the modified source code, change to the c:\driver\source directory and enter

nmake

This obtains its build instructions from the MAKEFILE file that was unzipped with the source files.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## Lotus 1-2-3 2.3 Driver

### Lotus 1-2-3 2.3 Driver Installation

1.	Insert the S3 DOS Drivers diskette into the appropriate diskette drive and change the current drive/directory path to the one which contains Lotus 1-2-3.

2.	Copy the following files from the \LOTUS sub-directory on the diskette to the directory which contains Lotus 1-2-3:

123txt25.drv
	123txt43.drv
	123d768.drv
	123s768.drv
	123d600.drv
	123s600.drv

4.	Type INSTALL and press Enter to run the Lotus 1-2-3 install program.

5.	Press Enter to continue.

You will be presented with several menu choices. Follow the directions on the screen and perform the following steps:

a.	Select "Change Selected Equipment".

b.	Select "Modify the Current Driver Set".

c.	Select "Text Display".

e.	Select "Graphics Display".

f.	Select the desired S3 text display driver. Do not select the drivers for dual display. This 	configuration is not supported by your computer.

g.	Press Esc to return to the previous menu.

h.	Select "Save the Current Drive Set".

i.	Press Enter to continue.

### Building the Lotus 1-2-3 2.3 Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source\lotus.

Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\lotus c:\driver\source\lotus

Create a subdirectory named \userdisk under the directory that contains the source code. The built driver will be placed in this subdirectory.

Use PKUNZIP to install the library files that are included in the file S3LIB.ZIP. These library functions are used by the Lotus driver and the makefile for the driver will need to be modified to contain the correct subdirectory for the library.

The source code can now be modified as desired. To build the executable code from the modified source code, run the BUILD.BAT batch file.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## Microsoft Word 5.0 Driver

### Microsoft Word 5.0 Driver Installation

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2.	Change the current drive/directory path to the one which contains Microsoft Word.

3.	Save a copy of SCREEN.VID by renaming it. Copy the SCREEN.VID file from the \MSWORD sub-directory on the diskette to the directory which contains Microsoft Word.

4.	Run Microsoft Word.

5.	Word uses several different display resolutions and character sizes to offer a wide variety of visible characters. Several new modes are now available for use, and the user may choose from a list by pressing the Esc key, the O key, then moving to the "DISPLAY MODE" entry with the arrow keys. Press the F1 key to display all of the available resolution choices. If necessary, press the Page Down key to display more resolutions. The new graphics modes are the entries for 37, 38, 42, 48, 54, 75 and 96 lines. The new text modes are the 25 and 43 lines (132 columns) . Select the resolution by moving the highlight and press Enter.

6.	After the display option is selected, Word will use the new resolution.

### Building the Microsoft Word 5.0 Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source\msword. Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\msword c:\driver\source\msword

Create a subdirectory named \userdisk under the directory that contains the source code. The built driver will be placed in this subdirectory.

Use PKUNZIP to install the library files that are included in the file S3LIB.ZIP. These library functions are used by the Lotus driver and the makefile for the driver will need to be modified to contain the correct subdirectory for the library.

The source code can now be modified as desired. To build the executable code from the modified source code, run the BUILD.BAT batch file.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

## WordPerfect 5.1 Driver

### Word Perfect 5.1 Driver Installation

1.	Insert the S3 DOS Drivers diskette in the appropriate diskette drive.

2.	Change the current drive/directory path to the one which contains Word Perfect v5.1.

3.	Copy the following files from the \WORDPERF subdirectory on the floppy disk to the directory on the hard disk where Word Perfect is installed:

WPGRAPH.VRS
WPTEXT.VRS

4.	Run Word Perfect.

5.	Word Perfect allows you to select from a list of graphics and/or text drivers from the setup screen. Press Shift-F1 to go to the setup screen, choose "Display Option", then choose either "Text" or "Graphics Display". You will be presented with several adapters to choose from, including S3 86C911 (Carrera) Text and Graphics entries. Select "Graphics" and choices for 1024x768 and 800x600 will be presented. Selecting "Text" will give you 132x25 and 132x43. Choose the appropriate resolution for Text and/or Graphics and continue to use Word Perfect as before.

### Building the WordPerfect 5.1 Driver

To build, modify or update the driver source code (which must be purchased separately), the following software utilities are required:

·	An ASCII text editor

·	Microsoft Macro Assembler (MASM), version 5.1 or later

·	Microsoft Linker (LINK), version 3.65 or later

·	Microsoft Library Utility (LIB), version 3.11 or later

·	Microsoft NMAKE utility, version 1.13 or later

Create a directory on the hard disk to hold the source code files, e.g., c:\driver\source\wp51. Unzip the source files on the "DOS and CAD Drivers Source Code" diskette and copy them into the appropriate directory, e.g.,

pkunzip b:\wordperf c:\driver\source\wp51

Create a subdirectory named \userdisk under the directory that contains the source code. The built driver will be placed in this subdirectory.

Use PKUNZIP to install the library files that are included in the file S3LIB.ZIP. These library functions are used by the Lotus driver and the makefile for the driver will need to be modified to contain the correct subdirectory for the library.

The source code can now be modified as desired. To build the executable code from the modified source code, run the BUILD.BAT batch file.

Note: Several warning messages may appear while building certain source modules. These warning messages should be ignored. However, there should be no "Error" or "Fatal" messages from the assembler, linker or make utility.

# S3 OS/2 Drivers

Note: The installation procedure has changed significantly from that used for the previous release of the S3 drivers. In particular, the installation must now be done from a floppy disk. The previous method of copying files to the hard disk before running the installation program cannot be used.

## Installation Considerations

1.	The installation process assumes that the user knows the maximum vertical refresh rate as well as the maximum resolution of the monitor.

2.	When installing the S3 driver over a non-S3 driver (e.g., SVGA), the standard IBM VGA driver must installed first. This is not required if an S3 driver is already installed.

3.	The S3 driver will not install unless an S3 chip-based card is installed in the machine at the time of driver installation.

4.	When installing IBM's VGA driver over an S3 driver, you must manually remove the line DEVICE=C:\OS2\MDOS\VENH.SYS from your CONFIG.SYS file. (If OS/2 is not installed on drive C:, use DEVICE=x:\OS2..., where x: is the drive on which you installed OS/2.)

5.	If S3 drivers (1.2, 1.1, or 1.0) have previously been installed, rebooting OS/2 after installing OS/2 2.1 will result in an error message (SYS1719) for the file \OS2\MDOS\VENH.SYS. Simply press "Enter" and continue to install the S3 drivers.

6.	These drivers are for the S3 86C911/924 and 86C80X/928 chips with  a AT&T 491-compatible RAMDAC and the 86C928 with a Brooktree Bt485 RAMDAC.

7.	The REXX language program must be installed to run S3's install utility program.

## Installation Instructions

1.	Make sure you have all open data files saved before installing the S3 drivers.

2.	Make sure all Seamless Windows applications are closed. (Seamless applications are windows applications that run on top of the PM Desktop.)

3.	Open an OS/2 command prompt (windowed or full screen)

4.	Insert driver diskette #1 into the floppy drive.

5.	Change to the floppy drive (A: or B:) into which you have inserted the driver diskette.

6.	Type S3INSTL and press "Enter".

7.	When prompted by the Display Driver Install dialog, select Primary Display and then click the OK button.

8.	Select the S3 801/805/911/924/928 Evaluation Boards option from the Primary Display Adapter Type dialog.

9.	When the Select Display Resolution dialog appears, select the desired resolution and scan rate.  Be careful not to choose a resolution or scan rate higher than your monitor supports.

10.	The Source Directory dialog will be displayed.  Make sure the source directory is set to the location you are installing from and select Install.

11.	The Installation Program may pause and display prompts to overwrite certain files. Please answer YES for each file.

12.	When the installation is done with diskette #1, insert diskette #2 and continue installing the S3 drivers.

13.	Once the installation is complete, you will be returned to the Display Driver Install main menu.  Select Cancel to exit the installation program.

14.	After you select Cancel, you will be given a message stating that you "Must Shutdown and Restart your system for the changes to take effect".  Select OK to continue back to the OS/2 command prompt.

15.	At this point you must shut down the system and reboot.  After you reboot your system will come up with your newly installed driver.

# OS/2 2.0 MINI PORT OEM Guide

## Introduction

S3 Incorporated recognizes the requirement for our customers to be able to customize OS/2 to support various hardware configurations. The information in this document enables OEMs to make customizations to the OS/2 driver. Customization examples are:

·	Modification of the clock parameters so that an alternate clock chip can be utilized.

·	Modification of CRTC timings for custom monitor support.

·	Support for various refresh rates

·	Support for an alternate RAMDAC

·	Modification of the DSPINSTL scripts.

Whatever the need, as long as it is satisfied by sequences of PORT I/O, the Mini Port feature of the S3 drivers will meet the requirement.

The Mini Port feature is a modetbls.asm file and a set of commands that can be modified by our OEM customers to create BVHENH.DLL. The mechanism for choosing between options is the DSPINSTL.EXE program.

The MiniPort is a user definable resource in OS/2 that is modified with any editor, assembled with Microsoft MASM 5.1 and then put together with the Resource Compiler supplied with OS/2. At initialization time, the BVH will read and interpret the tables in the process of setting the mode as requested by the PM driver.

## Mode Tables1

The modetbls.asm file contains tables that are interpreted by the BVH when setting a mode or when returning to VGA. The OPERATIONS available to the interpreter are:

·	OB 	out byte

·	OBC	out x bytes

·	OW	out word

·	OWC	out x words

·	OBRMW	out byte read/modify/write

·	IB	in byte

·	IBC	in x bytes

·	IW	in word

·	IWC	in x words

·	ATTROUT	out x bytes to the attribute controller

·	INDEXOUT	out x bytes to any indexed controller

·	NOP	no operation

·	VBLANK	wait x units of time

·	EOD	end of data

The following module is a sample of modetbls.asm. It is included to illustrate the structure of the mode tables and the usage of the command sequence. It does not mean that the following sample is currently being used. In addition, the sample only includes the 800 x 600 x 8 table structures.

;---------------------------------------------------------------------------

;  Mode table structures for BVHENH resources

;

; Copyright (c) Leonard Berk Systems Software Inc., 1992

; Copyright (c) S3, Incorporated, 1992.

; All rights reserved.

;

; Release 1.01 October 29, 1992

;---------------------------------------------------------------------------

;---------------------------------------------------------------------------

; This file, as supplied, builds an OS/2 user-defined binary resource which
; instructs the S3 enhanced mode BVH how to set each supported high-
; resolution mode on the S3 86C911/924/801 Evaluation Boards.  If setting
; these modes on your company's hardware requires different values in some
; registers, or a different algorithm, you must modify the COMMAND STREAM
; tables below.

;

; Any given build of the BVHENH uses five of the command stream tables:

; one each for 1024x768x8, 800x600x8, 1024x768x4, 800x600x4, and Reset_VGA.
; As supplied, this file permits the OEM to select among nine different sets
; of command stream tables, according to the frequency specifications of the
; target video monitor.  Each set of command stream tables is defined by a
; MODE TABLE structure.  You must select a particular mode table for a given
; build of the BVH.

; To select a mode table, enter its index into the usModeTableIndex field of the HEADER structure. At init time, the BVH will load the five command tables specified in this structure.

;

; If you must modify the values in the mode table structures, DO NOT modify
; the placement of the resolutions within the mode table.  Within any mode
; table, the first command stream pointed to must set 1024x768x8, the second
; 800x600x8, etc.

;

; Limitations:  Keep in mind that the BVH storage buffer for command stream
; tables is currently limited to 1000 words.  A set of five command stream
; tables, as defined here, occupies about 750 words. A set of command tables
; longer than 1000 words will produce unpredictable behavior in the BVH.

;---------------------------------------------------------------------------

;---------------------------------------------------------------------------

TABLE   SEGMENT

ASSUME cs:TABLE,ds:TABLE,ss:TABLE

ORG 0

;---------------------------------------------------------------------------

HEADER          struc

usResLength	dw	?	; Length of the resource in bytes

usModeTableIndex	dw	?	; Index of the mode table for this driver

usSizeModeTable	dw	?	; Length of a ModeTable in bytes

pusFirstTable	dw	?	; Offset of first mode table

HEADER          ends

MODETABLE   struc         ;

pusMode0	dw	?	; offset to the COMMAND STREAM table for 				;   1024x768x8

usLenTblMode0	dw	?	; length in bytes of the preceding table

pusMode1	dw	?	; offset to the COMMAND STREAM table for 				;   800x600x8

usLenTblMode1	dw	?	; length in bytes of the preceding table

pusMode2	dw	?	; offset to the COMMAND STREAM table for 				;   1024x768x4

usLenTblMode2	dw	?	; length in bytes of the preceding table

pusMode3	dw	?	; offset to the COMMAND STREAM table for 				;   800x600x4

usLenTblMode3	dw	?	; length in bytes of the preceding table

pusReset	dw	?	; offset to the COMMAND STREAM table for 				;   ResetVGA

usLenTblReset	dw	?	; length in bytes of the preceding table

MODETABLE       ends

;---------------------------------------------------------------------------

; The ResourceHeader must always be at the top of the table

;--------------------------------------------------------------------------------------------------

ResourceHeader    HEADER  <EndResource, 8, 20, ausModeTable0>

;---------------------------------------------------------------------------

; MODETABLEs begin here.  Each is a set of offsets to the five command

; streams to be used for a given ModeTable index, with each offset

; followed by the length of the associated table (in bytes)

;---------------------------------------------------------------------------

T1  =  Reset_VGA	; We'll use these aliases to keep the

T2  =  R800x600x8_56hz	; MODETABLE structures within a

T3  =  R800x600x8_60hz	; reasonable line length

T4  =  R800x600x8_72hz

T5  =  R800x600x4_56hz

T6  =  R800x600x4_60hz

T7  =  R800x600x4_72hz

T8  =  R1024x768x8_43hz

T9  =  R1024x768x8_60hz

T10 =  R1024x768x8_72hz

T11 =  R1024x768x4_43hz

T12 =  R1024x768x4_60hz

T13 =  R1024x768x4_72hz

T14 =  EndResource

ausModeTable0   MODETABLE  <R1024x768x8_43hz,T9-T8,R800x600x8_56hz,T3-

T2,R1024x768x4_43hz,T12-T11,R800x600x4_56hz,T6-T5,Reset_VGA,T2-T1>

ausModeTable1   MODETABLE  <R1024x768x8_43hz,T9-T8,R800x600x8_60hz,T4-

T3,R1024x768x4_43hz,T12-T11,R800x600x4_60hz,T7-T6,Reset_VGA,T2-T1>

ausModeTable2   MODETABLE  <R1024x768x8_43hz,T9-T8,R800x600x8_72hz,T5-

T4,R1024x768x4_43hz,T12-T11,R800x600x4_72hz,T8-T7,Reset_VGA,T2-T1>

ausModeTable3   MODETABLE  <R1024x768x8_60hz,T10-T9,R800x600x8_56hz,T3-

T2,R1024x768x4_60hz,T13-T12,R800x600x4_56hz,T6-T5,Reset_VGA,T2-T1>

ausModeTable4   MODETABLE  <R1024x768x8_60hz,T10-T9,R800x600x8_60hz,T4-

T3,R1024x768x4_60hz,T13-T12,R800x600x4_60hz,T7-T6,Reset_VGA,T2-T1>

ausModeTable5   MODETABLE  <R1024x768x8_60hz,T10-T9,R800x600x8_72hz,T5-

T4,R1024x768x4_60hz,T13-T12,R800x600x4_72hz,T8-T7,Reset_VGA,T2-T1>

ausModeTable6   MODETABLE  <R1024x768x8_72hz,T11-T10,R800x600x8_56hz,T3-

T2,R1024x768x4_72hz,T14-T13,R800x600x4_56hz,T6-T5,Reset_VGA,T2-T1>

ausModeTable7   MODETABLE  <R1024x768x8_72hz,T11-T10,R800x600x8_60hz,T4-

T3,R1024x768x4_72hz,T14-T13,R800x600x4_60hz,T7-T6,Reset_VGA,T2-T1>

ausModeTable8   MODETABLE  <R1024x768x8_72hz,T11-T10,R800x600x8_72hz,T5-

T4,R1024x768x4_72hz,T14-T13,R800x600x4_72hz,T8-T7,Reset_VGA,T2-T1>

;---------------------------------------------------------------------------

; Command stream tables begin here.

;---------------------------------------------------------------------------

;=========================================================================

; TABLES TO SET THE MODE OF THE S3 86C911/924/801/805 IN S3 EVALUATION
; BOARDS

;=========================================================================

; Define command flags

COUNT	equ 01h

WORDOP	equ 02h

RMW		equ 04h

INPUT		equ 08h

; Define basic commands

INOUT		equ 10h

LOOPOP	equ 20h

VBLANK	equ (40h+COUNT)

NULLOP	equ 80h

EOD		equ 0ffh

; Define output commands

OB		equ (INOUT)

OBC		equ (INOUT+COUNT)

OW		equ (INOUT+WORDOP)

OWC		equ (INOUT+COUNT+WORDOP)

OBRMW	equ (INOUT+RMW)

; Define input commands

IB		equ (INOUT+INPUT)

IBC		equ (INOUT+INPUT+COUNT)

IW		equ (INOUT+INPUT+WORDOP)

IWC		equ (INOUT+INPUT+COUNT+WORDOP)

; Define loop commands

ATTROUT	equ (LOOPOP+02h)

INDEXOUT	equ (LOOPOP+01h)

;=========================================================================

; RESET VGA Table Structure

;=========================================================================

Reset_VGA label word

dw OB,	3c4h, 01h	; Sequencer register 1

dw OBRMW,	3c5h, 0dfh, 20h	; Blank Screen

dw VBLANK, 06h	; Wait

dw OW,	3d4h, 0a039h	; Unlock

dw OW,	4ae8h, 02h	; Set VGA

dw OW,	3d4h, 4838h	; Unlock S3 regs

dw OWC,	3d4h, 05h

dw         0042h, 0045h, 0c40h, 0760h, 005eh

dw OB,	3d4h, 3ah	; Set S3R0A index

dw OBRMW,	3d5h, 0c7h, 00h	; Clear

dw OB,	3d4h, 31h	; Set S3R1 Index

dw OBRMW,	3d5h, 85h, 00h	; Maintain 16 bit if set

dw OW,	3d4h,      0032h	; S3R2 Clear

dw OB,	3d4h,      34h	; S3R4 Index

dw OBRMW,	3d5h,      0ech,0	; Clear selected bits

dw OWC,	3d4h,      0ah

dw         5a3bh,103ch,0050h,3854h,005dh

dw         8061h,0a162h,0858h,0033h,0043h

dw OW,	3d4h,      0038h	; Lock

dw OW,	3d4h,      0039h	; Lock

dw VBLANK, 0ah

dw OB,	3c4h,      01h	; Sequencer 1

dw OBRMW,	3c5h,       dfh,00h	; Set display on

dw EOD

;================================================================

;800x600x8 Table Structures

;

;800x600x8 56hz

;================================================================

R800x600x8_56hz label word

dw OW,	3c4h,          2101h	; Set screen Blank

dw VBLANK, 02h

dw OW,	3c4h,          1000h	; Async Reset

dw OWC,	3c4h,          04h

dw         2101h,0f02h,0003h,0e04h	; Sequencer

dw OW,	3d4h,          4838h	; Unlock

dw OW,	3d4h,          0a039h	; Unlock

dw OW,	3d4h,          0011h	; Unlock CRTC

dw INDEXOUT,3d4h,          19h,0

dw         80h,63h,64h,83h,6bh,00h,8eh,0f0h,80h,60h

dw         00h,00h,00h,00h,0ffh,00h,66h,8ch,57h,80h

dw         80h,63h,80h,0e3h,0ffh

dw IB,	3dah	; Set to use the ATC

dw ATTROUT, 3c0h,          15h,0

dw         00h,01h,02h,03h,04h,05h,14h,07h,38h,39h

dw         3ah,3bh,3ch,3dh,3eh,3fh,01h,00h,0fh,00h

dw         00h

dw OW,	3c4h,          0300h	; Start the Sequencer

dw INDEXOUT,3ceh,     09h,0

dw         00h,00h,00h,00h,00h

dw         00h,05h,0fh,0ffh

dw IB,	3dah	; Reset the ATC index

dw OB,	3c0h,          20h	; Enable the palette

dw OWC,	3d4h,          0eh

dw	0642h,0140h,0f60h,005eh,7b3bh

dw	913ch,0050h,1854h,005dh,8061h

dw	0c962h,0858h,0033h,0043h

dw OB,	3d4h,          31h

dw OBRMW,	3d5h,          0e4h,08h

dw OB,	3d4h,          3ah

dw OBRMW,	3d5h,          0e2h,15h

dw OB,	3d4h,          32h

dw OBRMW,	3d5h,          0bfh,0

dw OW,	4ae8h,         07h

dw OB,	3c2h,      02fh

dw VBLANK, 0ah

dw OW,	3d4h,          0039h

dw OW,	3d4h,          0038h

dw OB,	3c4h,          01h

dw OBRMW, 3c5h,          0dfh,0	; Enable the screen

dw EOD

;================================================================

;800x600x8 60hz

;================================================================

R800x600x8_60hz label word

dw OW,	3c4h,      2101h	; Set screen Blank

dw VBLANK, 02h

dw OW,	3c4h,      1000h	; Async Reset

dw OWC,	3c4h,      04h

dw         2101h,0f02h,0003h,0e04h	; Sequencer

dw OW,	3d4h,      4838h	; Unlock

dw OW,	3d4h,      0a039h	; Unlock

dw OW,	3d4h,      0011h	; Unlock CRTC

dw INDEXOUT, 3d4h,      19h,0

dw         7fh,63h,64h,82h,6ah,1ah,74h,0f0h,00h,60h

dw         00h,00h,00h,00h,0ffh,00h,58h,8ch,57h,80h

dw         00h,57h,73h,0e3h,0ffh

dw IB,	3dah	; Set to use the ATC

dw ATTROUT, 3c0h,       15h,0

dw         00h,01h,02h,03h,04h,05h,14h,07h,38h,39h

dw         3ah,3bh,3ch,3dh,3eh,3fh,01h,00h,0fh,00h

dw         00h

dw OW,	3c4h,      0300h	; Start the Sequencer

dw INDEXOUT, 3ceh, 09h,0

dw         00h,00h,00h,00h,00h

dw         00h,05h,0fh,0ffh

dw IB,	3dah	; Reset the ATC index

dw OB,	3c0h,      20h	; Enable the palette

dw OWC,	3d4h,      0eh

dw         0242h,0140h,0f60h,005eh,7a3bh

dw         913ch,0050h,1854h,005dh,8061h

dw         0c962h,0858h,0033h,0043h

dw OB,	3d4h,      31h

dw OBRMW, 3d5h,      0e4h,08h

dw OB,	3d4h,      3ah

dw OBRMW, 3d5h,      0e2h,15h

dw OB,	3d4h,      32h

dw OBRMW, 3d5h,      0bfh,0

dw OW,	4ae8h,     07h

dw OB,	3c2h,      02fh

dw VBLANK, 0ah

dw OW,	3d4h,      0039h

dw OW,	3d4h,      0038h

dw OB,	3c4h,      01h

dw OBRMW, 3c5h,      0dfh,0	; Enable the screen

dw EOD

;================================================================

;800x600x8 72hz

;================================================================

R800x600x8_72hz label word

dw OW,	3c4h,      2101h	; Set screen Blank

dw VBLANK, 02h

dw OW,	3c4h,      1000h	; Async Reset

dw OWC,	3c4h,      04h

dw         2101h,0f02h,0003h,0e04h	; Sequencer

dw OW,	3d4h,      4838h	; Unlock

dw OW,	3d4h,      0a039h	; Unlock

dw OW,	3d4h,      0011h	; Unlock CRTC

dw INDEXOUT,	3d4h,      19h,0

dw         7dh,63h,64h,80h,6ch,1bh,98h,0f0h,00h,60h

dw         00h,00h,00h,00h,0ffh,00h,7ch,0a2h,57h,80h

dw         00h,57h,98h,0e3h,0ffh

dw IB,	3dah	; Set to use the ATC

dw ATTROUT, 3c0h,   15h,0

dw         00h,01h,02h,03h,04h,05h,14h,07h,38h,39h

dw         3ah,3bh,3ch,3dh,3eh,3fh,01h,00h,0fh,00h

dw         00h

dw OW,	3c4h,      0300h	; Start the Sequencer

dw INDEXOUT, 3ceh,  09h,0

dw         00h,00h,00h,00h,00h

dw         00h,05h,0fh,0ffh

dw IB,	3dah	; Reset the ATC index

dw OB,	3c0h,       20h	; Enable the palette

dw OWC,	3d4h,      0eh

dw         0442h,0140h,0f60h,005eh,793bh

dw         403ch,0050h,1854h,005dh,8061h

dw         0c962h,0858h,0033h,0043h

dw OB,	3d4h,      31h

dw OBRMW, 3d5h,      0e4h,09h

dw OB,	3d4h,      3ah

dw OBRMW, 3d5h,      0e2h,15h

dw OB,	3d4h,      32h

dw OBRMW, 3d5h,      0bfh,0

dw OW,	4ae8h,     07h

dw OB,	3c2h,      02fh

dw VBLANK, 0ah

dw OW,	3d4h,      0039h

dw OW,	3d4h,      0038h

dw OB,	3c4h,      01h

dw OBRMW, 3c5h,      0dfh,0	; Enable the screen

dw EOD

## Customization Example

In building this module, you will note that all samples are for S3 evaluation boards. The following is the best strategy for modification:

·	Make only minimal changes to the tables.

·	Only change the command sequence where necessary for hardware constraints.

·	Once that is done, build as many variations as needed to support the installation scripts that DSPINSTL will be using. The variation is chosen by modifying the line:

ResourceHeader  HEADER  <EndResource, 8, 20, ausModeTable0>

The 8 in this line picks variation 8, which in this sample is the 70 Hz modes.

For example, to make modifications to the clock parameters for the 60 Hz modes, use the following steps (the sample modetbls.asm the Mode Tables section will be used):

·	Locate the mode table number for the 60 Hz modes. Using the following information from modetbls.asm, the number is 4:

ausModeTable4   MODETABLE  <R1024x768x8_60hz,T10-T9,R800x600x8_60hz,T4-

T3,R1024x768x4_60hz,T13-T12,R800x600x4_60hz,T7-T6,Reset_VGA,T2-T1>

·	Modify the HEADER line to reflect variation 4:

ResourceHeader  HEADER  <EndResource, 4, 20, ausModeTable0>

·	The CR42 register contains the clock parameters. Thus, locate the following line in one of the 60 Hz mode tables:

dw    OWC,    3d4h,      0eh
dw    XX42h,0140h,0f60h,005eh,7a3bh

XX represents the desired clock byte value needed for CR42.

## Building Mechanics

The building procedure is straight forward but multistep.  You must start with the Mini Port diskette that contains the files referenced in this section.

The resulting file will ALWAYS be named BVHENH.DLL, so you must pack it to match the building scripts for dspinstl (use the OS/2 Pack utility). Currently, there are 3 variations that are used:

·	BVHENH0.DL@ - 800x600 56Hz, 1024x768 43Hz, 1280x1024 45Hz

·	BVHENH4.DL@ - 640x480 60Hz, 800x600 60Hz, 1024x768 60Hz

·	BVHENH8.DL@ - 640x480 72Hz, 800x600 72Hz, 1024x768 70Hz

To assemble the modetbls.asm file, type in the following at the command prompt:

make modetbls.

This must be done under DOS.

The following steps must be completed under OS/2:

1.	rc -r bvhenh.rc

2.	copy bvhenhnr.dll bvhenh.dll

3.	rc bvhenh.res bvhenh.dll

Care must be taken not to run while in the \os2\dll directory of OS/2.

## DSPINSTL Scripts

S3FULL.DSC is used by the installation to describe the available resolutions. The following is a listing of S3FULL.DSC:

* Title

"S3 801/805/911/924/928 Evaluation Boards"

* Action routine DLL

"S3INST.DLL"

* Type key

"OTHER"

* # additional parameters specific to SVGA

"1"

* Additional parameter (Chipset manufacturer = 3 for Tseng)

"0"

* # of chain elements

"36"

* chain element = key, general prompt, diskette prompt, vol label, resolution

* Configuration chain element - key set set by SVGA.DLL (eg, CXXXxYYYxCCC)

""

* General prompt

"S3 Evaluation Boards"

* diskette prompt

"S3 Display Driver Diskette 1"

* volume label

"DISP 1"

* no resolution for configuration

""

*----------------------------------

* resolution chain elements follow

* S3 805 GenDAC

*----------------------------------

* first resolution

"640x480x256F#512K#4hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256               60Hz"

"640x480x256F#512K#8hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256               72Hz"

* 2nd   resolution

"800x600x256F#1M#0hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               56Hz"

"800x600x256F#1M#4hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               60Hz"

"800x600x256F#1M#8hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               72Hz"

* 3rd   resolution

"1024x768x256F#1M#0hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256              43 interlaced"

"1024x768x256F#1M#4hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256             60Hz"

"1024x768x256F#1M#8hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256              70Hz"

* 4th   resolution

"1280x1024x256F#2M#0hz#GENDAC"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1280x1024x256            45 interlaced"

*----------------------------------

* resolution chain elements follow

* S3 801/805/928 with AT&T DAC

*----------------------------------

* first resolution

"640x480x256F#512K#4hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256               60Hz"

"640x480x256F#512K#8hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256               72Hz"

* 2nd   resolution

"800x600x256F#1M#0hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               56Hz"

"800x600x256F#1M#4hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               60Hz"

"800x600x256F#1M#8hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256               72Hz"

* 3rd   resolution

"1024x768x256F#1M#0hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256              43 interlaced"

"1024x768x256F#1M#4hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256              60Hz"

"1024x768x256F#1M#8hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256              70Hz"

* 4th   resolution

"1280x1024x256F#2M#0hz#80x"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1280x1024x256            45 interlaced"

*----------------------------------

* resolution chain elements follow

* S3 911/924

*----------------------------------

* first resolution

"640x480x256F#1M#4hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256              60Hz"

"640x480x256F#1M#8hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256              70Hz"

* 2nd resolution

"800x600x256F#1M#0hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256              56Hz"

"800x600x256F#1M#4hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256              60Hz"

"800x600x256F#1M#8hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256              72Hz"

* 3rd   resolution

"1024x768x256F#1M#0hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256             43 interlaced"

"1024x768x256F#1M#4hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256             60Hz"

"1024x768x256F#1M#8hz#911"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256             70Hz"

*----------------------------------

* resolution chain elements follow

* S3 928 with BrookTree DAC

*----------------------------------

* first resolution

"640x480x256F#1M#4hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256                 60Hz"

"640x480x256F#1M#8hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 640x480x256                 72Hz"

* 2nd   resolution

"800x600x256F#1M#0hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256                 56Hz"

"800x600x256F#1M#4hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256                 60Hz"

"800x600x256F#1M#8hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

" 800x600x256                 72Hz"

* 3rd   resolution

"1024x768x256F#1M#0hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256                43 interlaced"

"1024x768x256F#1M#4hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256                60Hz"

"1024x768x256F#1M#8hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1024x768x256                70Hz"

* 4th   resolution

"1280x1024x256F#2M#8hz#928"

""

"S3 Display Driver Diskette 2"

"DISP 2"

"1280x1024x256 ni          60Hz"

The next file S31KX8F8.DSP is one of 6 DSP files that direct the actual actions that DSPINSTL will take:

:TITLE

1024x768x256

:KEY

C1024x768x256F#1M#8hz#80x

:FILES :MODE=PRIMARY

BVHVGA.DL@   %BOOTDRIVE%:\OS2\DLL

BVH8_80X.DL@ %BOOTDRIVE%:\OS2\DLL

FONTS.FO@    %BOOTDRIVE%:\OS2\DLL

:FILES :MODE=PRIMARY :MODE=WINDOWS

S3W1K.DR@    %BOOTDRIVE%:\OS2\MDOS\WINOS2\SYSTEM

:FILES :MODE=PRIMARY :MODE=DOS

VVGA.SY@    %BOOTDRIVE%:\OS2\MDOS

VENH.SY@    %BOOTDRIVE%:\OS2\MDOS

Finally, the DSPINSTL.LOG file contains a description of the errors which may have occurred during installation.

# S3 SCO Open Desktop Drivers

## Preface

These Installation and Release Notes document the S3 86CXXX DDX for SCO Open Desktop. Use this product with the SCO X11R4 Server.

The software provided in this package enables the SCO X11R4 Server to be used with S3 86C911, 86C924, 86C801, 86C805, and 86C928 based graphics adapters.

### Contents

The S3 86CXXX DDX for SCO Open Desktop consists of the following:

·	The dynamically loadable DDX and grafinfo files

·	The VGA console driver patch for the 86C911 with ODT 1.1 (courtesy of SCO)

·	These Release and Installation Notes

### Software Requirements

This package can be installed on systems running Release 2.0 of Open Desktop or Release 1.1, Update G of Open Desktop and Release 4.1 of the SCO X11R4 Server.

Copyright Ó 1991-1993 XwareÔ All Rights Reserved. Reproduced with permission.

## Installation

This package installs on Open Desktop 2.0, or Open Desktop 1.1 with Update G and the X11 Release 4 Server EFS.

### Installation Procedure

Perform the following steps to install the S3 86CXXX DDX.

Warning: The console driver patch must be installed before you can install a S3 86C911-based display adapter into an Open Desktop 1.1 system. Therefore, you must install this software before installing the adapter in your system.

1.	Login as root.

2.	Execute custom from the command line by typing custom and pressing "Return" or from sysadmsh by making the following sysadmsh selection:

System->Software

3.	Choose Install, A New Product, and Entire Product. When you are prompted to insert the requested volume, insert the distribution diskette, then select Continue and press "Return".

4.	Eventually you will be prompted for the first volume again, select Continue and press "Return".

5.	The kernel will be rebuilt to include the console driver patch. You will be asked if you want to boot this kernel by default, type "Y" and press "Return". When you are asked if you want the kernel environment rebuilt, type "Y" and press "Return".

6.	Select Update and press "Return".

7.	When prompted, select S3 86C911, 86C924, 86C801, 86C805, or 86C928 and press "Return".

8.	When prompted, select the desired display resolution and press "Return".

9.	When prompted, select the appropriate monitor type and press "Return".

10.	If desired you may now specify a specific console multiscreen for the configuration or accept the defaults by pressing "Return".

11.	Select Accept and press "Return".

12.	Select Quit and press "Return".

13.	Select Save and press "Return".

14.	The system configuration will be updated and you will have to press "Return" again.

15.	You will return to custom and will need to press "Return" again.

16.	Select Quit and press "Return".

17.	When the system prompt returns type "init 0" and press "Return".

18.	You may now install your S3 86CXXX DDX.

19.	When prompted to press any key to reboot, power off your system.

NOTE:	You may now install an S3 86CXXX-based display adapter into an Open Desktop 1.1 system.

The console driver patch (if necessary) and the DDX are installed. Refer to your SCO documentation for information on how to invoke the X server.

## Release Notes

### Supported Display Modes

The following display modes are supported:

·	640x480 256 colors at 60 Hz non-interlaced

·	640x480 256 colors at 72 Hz non-interlaced

·	640x480 32K colors at 60 Hz non-interlaced

·	640x480 64K colors at 60 Hz non-interlaced

·	800x600 16 colors at 56 Hz non-interlaced

·	800x600 16 colors at 60 Hz non-interlaced

·	800x600 16 colors at 72 Hz non-interlaced

·	800x600 256 colors at 56 Hz non-interlaced

·	800x600 256 colors at 60 Hz non-interlaced

·	800x600 256 colors at 72 Hz non-interlaced

·	1024x768 16 colors at 43 Hz interlaced

·	1024x768 16 colors at 60 Hz non-interlaced

·	1024x768 16 colors at 70 Hz non-interlaced

·	1024x768 256 colors at 43 Hz interlaced

·	1024x768 256 colors at 60 Hz non-interlaced

·	1024x768 256 colors at 70 Hz non-interlaced

·	1280x960 16 colors at 43 Hz interlaced

·	1280x1024 16 colors at 43 Hz interlaced

·	1280x1024 256 colors at 43 Hz interlaced

Hardware Cursor Support

·	Supports a 64x64 hardware cursor in 16 and 256 color modes

·	Supports a 64x32 hardware cursor in 32K and 64K color modes

·	Supports a 64x64 software cursor in the 1280x1024 16 and 256 color mode

### Problem Fixed in Version 3.02

Monochrome VGA monitors can now be used with SCO Open Desktop and S3 drivers.

### Known Limitations

True Color and Direct Color Modes

The color of objects in the 32K and 64K color modes will change as the object is drawn. Some X clients do not support True Color or Direct Color visual types, these include but are not limited to: scocolor, scoterm, and scopaint.

Running xbench

The program “xbench” will fail to display the text “hello” in the complex1 test in the 32K and 64K color modes. This is a problem with xbench and can be corrected by editing the source file complex1.c and adding the XSetForeground() and XSetBackground() function calls between the XCreateGC() and the XClearArea() function calls to the complex1_bench() routine as follows:

subGC=XCreateGC(dpy,subwin,0L,NULL);
XSetForeground(dpy,subGC,black);
XSetBackground(dpy,subGC,white);
XClearArea(dpy,subwin,0,90,400,20,False);

Hardware Cursor

Some character based applications will program the VGA cursor address register directly. This results in the colors of the hardware cursor changing. The cursor colors can be recovered by moving the cursor across a window boundary to force the cursor shape to change.

15/16 Bits/Pixel Support

SCO is not supported in 15/16 bits/pixel modes when using a Brooktree Bt485 RAMDAC.

## Creating Grafinfo Files

### Introduction

This chapter describes how to create grafinfo files for graphics adapters that can be supported by the S3 86CXXX DDX. This chapter assumes the reader is familiar with Xsco grafinfo files and the graphics adapter to be supported. Refer to the SCO documentation for complete information about creating grafinfo files.

### Grafinfo Parameters Supported

The S3 86C911 DDX supports a number of grafinfo parameters used to configure the DDX for a given display mode. The following list summarizes the supported parameters.

XDRIVER	graphics adapter driver

VISUAL	visual type

PIXWIDTH	horizontal resolution in pixels

PIXHEIGHT	vertical resolution in pixels

DEPTH	pixel depth in bits

RGBBITS	maximum bits per red, green, or blue guns

REDMASK	red gun mask bits

GRNMASK	green gun mask bits

BLUMASK	blue gun mask bits

ADJCR13	indicated whether to auto adjust CR13

### Parameter Descriptions

The supported parameters are described in alphabetic order in the following paragraphs in detail.

Syntax:	ADJCR13 = value;

Description:	specifies whether CR13 is to be automatically adjusted for the selected pixel depth. A zero (0) indicated no and a one (1) indicate yes. This parameter is optional and need only be specified to override the default value of 1.

Example:	ADJCR13	= 1;

Syntax:	BLUMASK = value;

Description:	specifies the bits in the pixel that are used to drive the blue gun of the DAC in modes where DEPTH been specified as 16 bits per pixel. This parameter is optional and need only be specified to override the default value of 0x001F.

Example:	BLUMASK	= 0x001F;

Syntax:	DEPTH	= value;

Description:	specifies the pixel depth in bits. The depths supported are 4, 8, and 16 bits per pixel. This value is used to determine the default value used to initialize the 86C911 before the SetGraphics procedure is called.

Example:	DEPTH	= 8;

Syntax:	GRNMASK = value;

Description:	specifies the bits in the pixel that are used to drive the green gun of the DAC in modes where DEPTH been specified as 16 bits per pixel. This parameter is optional and need only be specified to override the default value of 0x03E0.

Example:	GRNMASK	= 0x03E0;

Syntax:	PIXHEIGHT	= value;

Description:	specifies the height in pixels for the display mode.

Example:	PIXHEIGHT	= 480;

Syntax:	PIXWIDTH	= value;

Description:	specifies the width in pixels for the display mode.

Example:	PIXWIDTH	= 640;

Syntax:	REDMASK = value;

Description:	specifies the bits in the pixel that are used to drive the red gun of the DAC in modes where DEPTH been specified as 16 bits per pixel. This parameter is optional and need only be specified to override the default value of 0x7C00.

Example:	REDMASK	= 0x07C00;

Syntax:	RGBBITS = value;

Description:	specifies the bits per gun in the “Pseudo Color” modes, otherwise it specifies the maximum bits per the red, green, or blue guns in the “True Color” modes. This parameter is optional and need only be specified to override the default value of 6 for the “Pseudo Color” modes or 5 for the “True Color” modes.

Example:	RGBBITS	= 6;

Syntax:	VISUAL	= value;

Description:	specifies the visual type for the display mode. The value specified must be “Pseudo Color” for 4 and 8 bit pixel depths and “True Color” for 16 bit pixel depths.

Example:	VISUAL	= “Pseudo Color”;

Syntax:	XDRIVER	= value;

Description:	specifies the graphics adapter driver that is loaded when the Xsco server starts. The value specified must be “s3c” for the S3 86CXXX DDX to be loaded when the server is invoked.

Example:	XDRIVER	= “s3c”;

### Grafinfo Procedures Supported

Grafinfo files allow procedure definitions that consist of I/O read and write operation to initialize the graphics adapter.

NOTE:	86CXXX refers to 86C911, 86C924, 86C801, 86C805, or 86C928.

SetGraphics

This procedure is called after the 86CXXX has been initialized with the default values for enhanced graphics modes. This allows the SetGraphics procedure to initialize only the registers that differ from the default values determined from the parameters in the DATA section, and where any OEM specific registers can be initialized. This procedure is optional and may be omitted if the default values are acceptable. The 86CXXX extended registers are unlocked, and the screen is blanked when this procedure is called.

SetText

This procedure is called after the 86CXXX has been reset to standard VGA modes. This procedure is optional and may be omitted if no OEM specific register need be reset. The 86CXXX extended registers are unlocked, and the screen is blanked when this procedure is called.

### Sample Grafinfo File

/*

* 	@(#)86c911.xgi		1.16 1/19/92

*

*	Copyright (C) Xware, 1991-1992.

*

* 	The information in this file is provided for the exclusive use

*	of the licensees of Xware. Such users have the right to use,

*	modify, and incorporate this code into other products for

*	purposes authorized by the license agreement provided they

*	include this notice and the associated copyright notice with

*	any such product.

*

*

* 	Copyright (C) The Santa Cruz Operation, 1991.

*

*	The information in this file is provided for the exclusive use

*	of the licensees of The Santa Cruz Operation, Inc. Such users

*	have the right to use, modify, and incorporate this code into

*	other products for purposes authorized by the license agreement

*	provided they include this notice and the associated copyright

*	notice with any such product. The information in this file is

*	provided “AS IS” without warranty.

*

*/

/*

* Supported Modes:

*

*		S3.86C911.VGA.640x480-256-60

*		S3.86C911.VGA.640x480-256-78

*		S3.86C911.VGA.640x480-32K-60

*		S3.86C911.VGA.640x480-64K-60

*		S3.86C911.VGA.800x600-16-56

*		S3.86C911.VGA.800x600-16-60

*		S3.86C911.VGA.800x600-16-76

*		S3.86C911.VGA.800x600-256-56

*		S3.86C911.VGA.800x600-256-60

*		S3.86C911.VGA.800x600-256-76

*		S3.86C911.VGA.1024x768-16-47

*		S3.86C911.VGA.1024x768-16-60

*		S3.86C911.VGA.1024x768-16-72

*		S3.86C911.VGA.1024x768-256-47

*		S3.86C911.VGA.1024x768-256-60

*		S3.86C911.VGA.1024x768-256-72

*		S3.86C911.VGA.1280x960-16-48

*		S3.86C911.VGA.1280x1024-16-46

*

*/

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 640x480-256-60 “640x480 256-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 640;		/* horz resolution in pixels				*/

PIXHEIGHT	= 480;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 640x480-256-78 “640x480 256-colors, 78HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 640;		/* horz resolution in pixels				*/

PIXHEIGHT	= 480;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x42); out(0x3D5, 0x0B);				/* set MODE-CTL	*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 640x480-32K-60 “640x480 32K-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “TrueColor”;

PIXWIDTH	= 640;		/* horz resolution in pixels				*/

PIXHEIGHT	= 480;		/* vert resolution in pixels				*/

DEPTH	= 16;			/* pixel depth in bits					*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 640x480-64K-60 “640x480 64K-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “TrueColor”;

PIXWIDTH	= 640;		/* horz resolution in pixels				*/

PIXHEIGHT	= 480;		/* vert resolution in pixels				*/

DEPTH	= 16;			/* pixel depth in bits					*/

RGBBITS	= 6; 			/* max bits per R, G, or B guns			*/

REDMASK 	= 0xF800;	/* red gun mask bits					*/

GRNMASK 	= 0x07E0;	/* green gun mask bits					*/

BLUMASK 	= 0x001F;	/* blue gun mask bits					*/

}

/*=====================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-16-56 “800x600 16-colors, 56 non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x06);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-16-60 “800x600 16-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x02);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-16-76 “800x600 16-colors, 76HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x04);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-256-56 “800x600 256-colors, 56 non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x06);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-256-60 “800x600 256-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x02);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 800x600-256-76 “800x600 256-colors, 76HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 800;		/* horz resolution in pixels				*/

PIXHEIGHT	= 600;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x7F; r1 = 0x63; r2 = 0x64; r3 = 0x82;

r4 = 0x66; r5 = 0x00; r6 = 0x8E; r7 = 0xF0;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x66; r17 = 0x8C; r18 = 0x57; r19 = 0x80;

r20 = 0x00; r21 = 0x63; r22 = 0x80; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x7A);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x04);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-16-47 “1024x768 16-colors, 47HZ interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x99; r1 = 0x7F; r2 = 0x7F; r3 = 0x9C;

r4 = 0x83; r5 = 0x19; r6 = 0x97; r7 = 0x1F;

r8 = 0x00; r9 = 0x40; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x81; r17 = 0x83; r18 = 0x7F; r19 = 0x80;

r20 = 0x00; r21 = 0x80; r22 = 0x96; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x94);			/* set S3R0B			*/

out(0x3D4, 0x3C); out(0x3D5, 0x60);			/* set S3R0C			*/

out(0x3C2, 0x2F);					/* set MISC 			*/

out(0x3D4, 0x42); out(0x3D5, 0x27);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*=====================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-16-60 “1024x768 16-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xA1; r1 = 0x7F; r2 = 0x80; r3 = 0x84;

r4 = 0x83; r5 = 0x94; r6 = 0x24; r7 = 0xFD;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x04; r17 = 0x8A; r18 = 0xFF; r19 = 0x80;

r20 = 0x60; r21 = 0x02; r22 = 0x22; r23 = 0xAB;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x9C);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x0D);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-16-72 “1024x768 16-colors, 72HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xA1; r1 = 0x7F; r2 = 0x80; r3 = 0x84;

r4 = 0x83; r5 = 0x94; r6 = 0x24; r7 = 0xFD;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x04; r17 = 0x8A; r18 = 0xFF; r19 = 0x80;

r20 = 0x60; r21 = 0x02; r22 = 0x22; r23 = 0xAB;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x9C);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x0E);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-256-47 “1024x768 256-colors, 47HZ interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0x99; r1 = 0x7F; r2 = 0x7F; r3 = 0x9C;

r4 = 0x83; r5 = 0x19; r6 = 0x97; r7 = 0x1F;

r8 = 0x00; r9 = 0x40; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x81; r17 = 0x83; r18 = 0x7F; r19 = 0x80;

r20 = 0x00; r21 = 0x80; r22 = 0x96; r23 = 0xE3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x94);			/* set S3R0B			*/

out(0x3D4, 0x3C); out(0x3D5, 0x60);			/* set S3R0C			*/

out(0x3C2, 0x2F);					/* set MISC 			*/

out(0x3D4, 0x42); out(0x3D5, 0x27);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-256-60 “1024x768 256-colors, 60HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xA1; r1 = 0x7F; r2 = 0x80; r3 = 0x84;

r4 = 0x83; r5 = 0x94; r6 = 0x24; r7 = 0xFD;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x04; r17 = 0x8A; r18 = 0xFF; r19 = 0x80;

r20 = 0x60; r21 = 0x02; r22 = 0x22; r23 = 0xAB;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x9C);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x0D);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1024x768-256-72 “1024x768 256-colors, 72HZ non-interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1024;		/* horz resolution in pixels				*/

PIXHEIGHT	= 768;		/* vert resolution in pixels				*/

DEPTH	= 8;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xA1; r1 = 0x7F; r2 = 0x80; r3 = 0x84;

r4 = 0x83; r5 = 0x94; r6 = 0x24; r7 = 0xFD;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0x00; r15 = 0x00;

r16 = 0x04; r17 = 0x8A; r18 = 0xFF; r19 = 0x80;

r20 = 0x60; r21 = 0x02; r22 = 0x22; r23 = 0xAB;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0x9C);			/* set S3R0B			*/

out(0x3D4, 0x42); out(0x3D5, 0x0E);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1280x960-16-48 “1280x960 16-colors, 48HZ interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1280;		/* horz resolution in pixels				*/

PIXHEIGHT	= 960;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xC0; r1 = 0x9F; r2 = 0x9F; r3 = 0x83;

r4 = 0xA4; r5 = 0x1F; r6 = 0xFC; r7 = 0x1F;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0xFA; r15 = 0xE8;

r16 = 0xE1; r17 = 0x85; r18 = 0xDF; r19 = 0x80;

r20 = 0x00; r21 = 0xE0; r22 = 0xFA; r23 = 0xA3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0xBB);			/* set S3R0B			*/

out(0x3D4, 0x3C); out(0x3D5, 0x60);			/* set S3R0C			*/

out(0x3C2, 0x2F);					/* set MISC 			*/

out(0x3D4, 0x42); out(0x3D5, 0x2E);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*=====================================================================*/

VENDOR S3 “S3”

MODEL 86C911 “86C911”

CLASS VGA “VGA”

MODE 1280x1024-16-46 “1280x1024 16-colors, 46HZ interlaced”

DATA

{

XDRIVER	= “s3c”;		/* S3 86C911 Carrera driver				*/

VISUAL	= “PseudoColor”;

PIXWIDTH	= 1280;		/* horz resolution in pixels				*/

PIXHEIGHT	= 1024;		/* vert resolution in pixels				*/

DEPTH	= 4;			/* pixel depth in bits					*/

}

PROCEDURE SetGraphics

{

out(0x3D4, 0x11); out(0x3D5, 0x00);			/* unlock CRTC			*/

r0 = 0xC0; r1 = 0x9F; r2 = 0x9F; r3 = 0x83;

r4 = 0xA4; r5 = 0x1F; r6 = 0x1A; r7 = 0xB2;

r8 = 0x00; r9 = 0x60; r10 = 0x00; r11 = 0x00;

r12 = 0x00; r13 = 0x00; r14 = 0xFA; r15 = 0xE8;

r16 = 0x01; r17 = 0x85; r18 = 0xFF; r19 = 0x80;

r20 = 0x00; r21 = 0x00; r22 = 0x19; r23 = 0xA3;

r24 = 0xFF;

bout(25,0x3D4,0x3D5);					/* set CR0-CR22		*/

out(0x3D4, 0x3B); out(0x3D5, 0xBB);			/* set S3R0B			*/

out(0x3D4, 0x3C); out(0x3D5, 0x60);			/* set S3R0C			*/

out(0x3C2, 0x2F);					/* set MISC 			*/

out(0x3D4, 0x42); out(0x3D5, 0x2E);			/* set MODE-CTL		*/

outw(0x4AE8, 0x0007);					/* set ADVFUNC			*/

}

/*============================================================================*/