INTRODUCTION

-----------------------------------------------------------------

The following table summarizes the major files on the video

drivers diskette for VGA 1024. Refer to these tables

to determine which drivers are available.

Video drivers diskette

---------------------------------------------------------------

Driver           Resolution     Colors    Notes

MMWD480.DRV      640x480        256       gamma version

NMMWD480.DRV     640x480        256       non-gamma version

VGA800.DRV       800x600        16

VGA1024.DRV      1024x768       16        does not operate

properly in Standard

mode, use in Enhanced

mode only

The 256-color palettes for the gamma and non-gamma drivers are

defined differently. The gamma drivers match the color scheme

used by the 16-color drivers. The non-gamma drivers use true

colors and may enhance the display on very high quality monitors.

We recommend using the gamma drivers.

Installing a Windows Video Driver

-----------------------------------------------------------------

Follow these instructions to install the Windows video drivers.

1.   Install Windows 3.1 with the standard VGA driver. Run

Windows, and check that Windows starts without any error

conditions. Then, exit Windows.

2.   Change to the Windows directory.

3.   Type:

setup

and press [Enter]

4.   Use the direction keys to highlight "Display," and press

[Enter].

5.   Use the direction keys to highlight "Other," and press

[Enter].

6.   When prompted for the display drivers diskette, insert the

video drivers diskette into Drive A and press [Enter].

A list of available drivers is displayed.

7.   Select a driver compatible with your monitor type, and

press [Enter].

.    Refer to your computer or video adapter documentation

and your monitor documentation for information about

monitor type requirements for video modes.

.    Drivers that require 512KB of video memory are

identified by the notation 512KB.

.    The 256-color palettes for the gamma and non-gamma

drivers are defined differently. The gamma drivers

match the color scheme used by the 16-color drivers.

The non-gamma drivers use true colors and may enhance

the display on very high quality drivers. We recommend

using the gamma drivers.

8.   Follow the prompts on the screen for inserting the Windows

diskettes and the video drivers diskette. When finished,

the SETUP program returns to MS-DOS.

9.   Run Windows. Windows will run in the new video mode and

display the resolution of the driver you selected. Verify

that the driver operates correctly.

If your Windows screen is not readable after installing the video

driver, exit Windows. Then, rerun the Windows SETUP program from

MS-DOS, and select a lower resolution. To exit Windows, press

[Ctrl+Alt+Del] to restart the computer.

Changing Windows Video Drivers

-----------------------------------------------------------------

After you determine that a driver operates correctly on your

system, you can select that driver by using the Windows SETUP

program from MS-DOS. Pick one of the VGA 1024 drivers.

You can also select a driver by using the Windows SETUP icon

from within Windows. Pick one of the VGA 1024 drivers. The

Windows SETUP program prompts you for the OEM display driver

disk if this diskette is not already in Drive A. At the prompt,

insert the video drivers diskette and press [Enter]. After

copying begins, Windows SETUP might again prompt for this

diskette as it searches for the WIN.CNF file. Enter the path:

<drive>\windows\system

where <drive> is the letter of the drive on which you installed

Windows.

The Windows 3.1 SETUP program's icon lets you restart Windows

without exiting Windows. Save your data files before doing an

automatic restart.