Tomcat Softworks, Lynwood, Ca

Contact: 71777,2564

Romlock Documentation

Date:  09Jul89

Lock and unlock software requires:

Hitachi CD-ROM drive:  CDR1503S, CDR3500 or CDR3600

(or 100% Hitachi compatible similar model)

Hitachi PC  bus interface card:  CDIFI3, CDIFI4, CDIFI4A, CDIF35A

CDIF25A, CDIF25A2

IBM Compatible PC, XT, AT or PS/2 25 or 30 class computer.

One to four drives at: 300, 200, 220, 320, 340 or 360 base

address

An application which does not reset the CD-ROM drive when it

starts.

Applications which use Microsoft Extensions generally will not

reset the drive after the MSCDEX.EXE is executed.

These files send a software lock command to all four drives (0-3)

at the address (LOCKxxx), and then exit to DOS:

LOCK300.EXE  LOCK200.EXE  LOCK220.EXE LOCK320.EXE  LOCK340.EXE

LOCK360.EXE

Select the file which matches the base address of your interface

card.  Copy the file from the distribution disk.  Rename it as

needed.

For example  LOCK.EXE

Power on your CD-ROM drive, insert the disc, and execute the lock

file by entering its name. (ie:  LOCK )   Test the operation by

pressing the Eject button (CDR3500/3600) or Open/Close button

(CDR1503S).  If the drive is locked, select the similar unlock

file, and execute it to verify unlock operation.

If lock did not work, the program address may not match the card

address.  You can determine the Port address in use on your

CD-ROM drive system by inspecting the file named CONFIG.SYS in

the root directory of your boot drive.  Change to the root

directory on your system, and enter:

TYPE CONFIG.SYS

There will generally be at least the following lines:

FILES=20

BUFFERS=20

LASTDRIVE=Z

DEVICE=C:\DEV\HITACHIA.SYS /D:MSCD000 /N:1 /P:360

The number which follows the /P: is the Port address.  If there

is no /P:, then the address used is 300.  In the example line

above, 360 is the port address in use.

If you want to automatically lock the drive when booting, add the

name of the lock file to the AUTOEXEC.BAT immediately after the

line containing the MSCDEX.EXE line.  The MSCDEX.EXE line should

be the first line, and LOCK.EXE should be on the second line for

maximum protection.

These files send a software unlock command to all four drives at

the address:

KCOL300.EXE  KCOL200.EXE  KCOL220.EXE  KCOL320.EXE  KCOL340.EXE

KCOL360.EXE

Kcol is lock spelled in reverse order.  Select the unlock file

which matches the base address used on the interface card and

lock file.  Copy it and rename it as needed.  If used on a system

available to the public, put it in some subdirectory to hide it,

or name it something you can remenber.  Otherwise, name it

UNLOCK.EXE.

The files should maintain the .EXE extension to operate.

Remember:  Software lock is cleared if the drive power is turned

OFF, and back ON, or by software reset or computer reset.  Even

if the LOCK command is executed in the AUTOEXEC.BAT file, there

will be a period of time where the disc is unprotected, and the

drive Eject or Open/Close switch will operate.

Some software will reset the drive when the application first

starts, and audio player software which uses Lock or has a Reset

function, will clear the software lock.  If an application

program recovers from a drive or program error by resetting the

drive or computer, that may clear the lock mode.

Software lock is provided on the drive for applications to use so

the application doesn't have to keep checking to see if the disc

has been changed by the user.

My locking program takes advantage of the feature to minimize

disc losses and program errors caused by attempting to read a

disc which was ejected by someone during trade shows and other

demos.

The locking and unlocking files are Freeware and may be copied

for use as needed.  As Freeware, the programs may not be sold.

Any duplication fees for a floppy disk containing these programs

may not exceed the fee currently charged by PC-SIG for a similar

disk.  I would appreciate notification if you distribute these

programs for a fee or if they become part of any CD-ROM disc.

The programs are user written software and are not approved or

supported by any Hitachi company or division.  You willingly

accept any and all risks involved in their use (or your inability

to use the programs), including any CD-ROM discs stolen by users

while the program is in use.

Be sure to use only the file which matches the address of your

interface card.  This will avoid problems caused by the program

"talking" to something other than the Hitachi drive.

The programs were created using Quick C.  Unlike my Forth locking

software, these programs do not check lock status, or make any

report that anything has happened when run.  You simply return to

the DOS prompt when the program finishes or times out.  If the

lock program is used with the tray open or no cartridge

installed, and the power is on, the disc will gemerally be locked

when it is inserted.

The programs are hardware direct.  Only the PC bus interface

cards combined with the Hitachi CD-ROM bus drives are supported.

These hardware direct programs are smaller than a similar

extensions based program.  Very fast computers may overrun the

timeout loops, or the lock/unlock command may not operate, or

only sometimes work.

The locking software is based on my hardware direct audio CD

player software, "Forthplay" created with F83 Forth, and

"Romplay" in Gwbasic.

This Freeware is dedicated to all the other people who create

free software for any computer system.   Thanks...  Roscoe