SHOWHPGL	A viewing program for HP-GL and HP-GL/2 plotting programs.

Version 1.9

Introduction

This program is designed to preview HP-GL and HP-GL/2 plotting programs

on the console screen of an IBM PC or compatible computer.  It

impliments 47 of 97 HP-GL or HP-GL/2 instruction.  Of the 97

instruction, 22 either have no meaning for this type of program or

are otherwise unrelated to drawing.  Four instructions are included to

provide backward compatibility with the HP9872 plotter.  These instructions

are interpreted as NOP's.

It is written in Microsoft C, version 6.0.  It requires MS-DOS

version 3.0 or above to run.  This program is meant to be used as a

preview tools for HP-GL files.  It is as accurate as it could be made

with the limited information and resources available at the time it

was written.  Known bugs or deficiencies are listed after the

instruction set summary.

Version 1.9 adds solid fills to RR and RA (relative and absolute filled

rectangles), and fixes a bug which allowed drawing outside the hard clip

border when using IW (windowing).

Revision History

Version 1.9	17 August, 1992

- Added solid fill types to RA and RR.

- Added transparent data (TD) instruction.

- Fixed bug allowing drawing outside P1/P2 border using IW.

Version 1.8 	8 August, 1992

- The LB command would not draw labels with more non-printing characters

than printing characters.  This caused labels which used embedded

backspaces for positioning to not be drawn.  Fixed.

- The LB command would not terminate properly at the end-of-file without

a label terminator present.  Fixed.

- Relative text size on the CRT increased as paper size enlarged.

'Should have decreased.  Fixed.

- IW clipping window incorrectly placed when P1/P2 (the corner

hard clip limits) were not lower-left and upper-right, respectively.

This resulted in no visible lines being drawn.  Fixed.  It also failed

to restore the correct clipping when turned off if scaling was on.

Fixed.

- DR drew characters in the wrong direction when P1/P2 were exchanged.

Fixed.

- HP-GL/2 defaults to SI (absolute character sizes) at initialization

(IN) or when issuing the DF (default) instruction, as does the

DraftPro plotter.  However, the HP7475 and HP7470 plotters specify

SR (relative character sizes for these cases.  Two switches were

added (-70 and -75) to select relative character scaling as the

default condition.

- XT and YT (tick drawing commands: not HP-GL/2 instructions) were able

to draw ticks outside the P1/P2 hard clip border.  Fixed.  Also,

tick length was not rescaling when P1/P2 changed unless TL was

executed after P1/P2 changed.  Fixed

- Type 1 scaling (isotropic, with no left and bottom parameters

specified) did not result in symmetric spacing on the axis which

was lengthened to achieve isotropic scaling.  Fixed.

- A single pixel line would terminate the program with an integer

divide overflow. Fixed.

- Filled rectangles didn't work properly if scaling was on.  Fixed.

- Operation of IR not clearly specified for the case of P1/P2 being

exchanged.  IR as implimented is based on hard clip borders and

assumes, for reference, P1 is the lower left corner and P2 is the

upper left corner.

Version 1.6	25 July, 1992

- Fixed bug in graphix.lib preventing labeling from having more non-

printing characters than printing chars and still be drawn.

- Relative character sizes increased as larger paper sizes were selected.

They should have decreased due to the necessary reduction in size to

fit the drawing to the screen.

- DR set the wrong character direction when P1/P2 (the hard clip corner

points) were exchanged.

- IW set the wrong window coordinates when P1/P2 were exchanged.  The

result was no drawing on the screen.

Version 1.5	30 December, 1991

- Added debugging flag.  Error messages regarding the source file are now

suppressed.  The -db flag turns them on.  Error messages are printed to

stdout and may be redirected to a file.

- Added four instructions for HP9872 plotter backward compatibility.

- re-wrote the graphics adapter identification code to make maintaining

the code easier.

- Added PE (polyline encoded) instruction.

Version 1.0	3 September, 1991

- Original release.

The P1/P2 Exchange Problem

There is an issue of semantics which arises for the case of P1/P2

being exchanged.  On a plotter, entering the P1 and P2 points is a

physical process whereby the operator mechanically places the cursor

at the desired corner points.  When one places P1 in the upper left

corner and P2 in the lower left corner of the page, the logical

coordinate system and the physical coordinate system of the plotter

become reflected and inverted.  The plotter defines P1 as the lower

left corner of the page and P2 as the upper right corner of the page.

Thus, if you move the pen to the physical location P1 or P2

coordinates (using the control panel), the plotter moves the pen to

the upper right or lower left corner, respectively.  However, if you

told the plotter to move to the coordinates of P1 or P2, it would

move to a point opposite of the respective location.  For example, if

one sets P1 = (10250, 7479) and P2 = (250, 279), issuing "PU250,279;"

would move the plotter to the upper right corner of the page, even

though the physical P2 location is in the lower left corner of the

page.

The plotter is being forced to perform a translation from physical

coordinates into logical coordinates.  By choosing P1 = (10250, 7479)

and P2 = (250, 279), we have inverted and reflected the logical

coordinate system.  Thus, moving the pen to the logical

coordinates of P1 (we are not commanding the plotter to move to the

physical location of P1) moves the pen to the lower left corner of

the page.  Our choice of coordinate values for P1 and P2 has placed

larger numerical values in the lower left corner of the page and

smaller values in the upper right corner.

What does this mean? If you examine the display for program

SHOW_TST.DPF, two of the four frames show P2 in the lower left corner

of the frame.  These frames have P1 and P2 exchanged.  If you examine

the program, the coordinate values given for point P2 are the values

of the P1 coordinate.  The labels are intended to show where the

physical location of P2 would be if you could display it.  However,

the P2 logical coordinate is always in the upper right corner, and

the P1 logical coordinate is always in the lower left corner.  Thus,

wer are forced to appear to mislabel the graph in order to show the

physical P1 and P2 coordinate locations.

Usage

SHOWHPGL -[video][paper][plotter][debug]  [filename]

where

filename	optional filename for HP-GL source file.  If a

filename is not supplied, standard input is

assumed.

video		option to force usage of a particular video

mode.  Values are: c, e, v, for CGA, EGA, or

VGA.

'c' will force CGA 640 x 200 monochrome mode,

'e' will force EGA 640 x 350 16 color mode,

'v' will force VGA 640 x 480 16 color mode.

Default mode is autodetect and select

highest resolutions standard mode available.

No SVGA modes are supported.

paper		One of the following standard paper sizes:

A, B, C, D, E, or A4, A3, A2, A1, A0 .

Default size is A. Case is important!!

A0 is a paper size option: a0 is unknown.

plotter		Default emulation is HPGL/2.

'dp'  will select DraftPro emulation.

'70'  will select HP7470 emulation.

'75'  will select HP7475 emulation.

The HP 7470/7475 have the plotter units coordinate

origin in the lower left corner.  The DraftPro

uses the center of the page as the origin.  Note also

that the 7470 and 7475 emulations do NOT limit the

number of pens available to 2 and 6, respectively.  Pen

colors are limited to those available on the graphics

mode you are running (i.e., CGA, EGA, or VGA).

debug		Default is to silently pass errors.  -db will

turn on error output to stdout.

The user may also set the environment variable HP-GL to a default set

of switches, i.e.,

set hpgl=dpA3

specifies default options of DraftPro mode and A3 size paper.

If command line switches are present, the environment settings are

ignored.  Multiple paper size selections will cause the program to

abort.  Multiple video mode selections will default to the highest

resolution autodetected mode.

The switch character is defined as '-'.   It is hard coded into the

program.  Directory path names may include either or both '\' and '/'

directory separators.  This program has been tested under MS-DOS 3.20

and MS-DOS 5.0 .

All video graphics functions draw using the video BIOS.  No direct

screen I/O is used.  Additional BIOS supported graphics modes may be

supported by defining the appropriate structures in graphix.h and the

macros in graph.h and making appropriate changes to the mode lists in

autogrph.c and showhpgl.c .

When the debugging flag is turned on, this program generates diagnostic

output text strings when illegal or unsupported instructions are

encountered.  THESE STRINGS APPEAR IN THE GRAPHICS DISPLAY AND MAY

INTERFERE WITH THE DRAWING IF SUFFICIENT ERRORS OCCUR.  If you expect

large numbers of errors and wish to retain a listing of the output,

redirect any generated output to a file.

Note that this program DOES NOT restore the video mode which was

in use prior to calling this program.  Although the EGA and VGA

display systems are capable of better than 80 columns and 25 rows on

the screen, they do not appear as supported IBM BIOS video modes.

Determination of nonstandard video mode parameters was beyond the

scope of this program.  If you want to restore a recognized video mode

at the end of program execution, add the following code to showhpgl.c just

before the end of main ():

twait (infile, 50);

graphics_off ();

This will introduce a 50 second delay before restoring the video mode.

Shorter delays can be had by changing the value 50 to a more reasonable

number.  If the initial video mode was not a BIOS standard video mode,

graphics_off () may not be able to restore it properly.

Compilation

The entire package is compiled as follows:

cl showhpgl.c hpgl1.c hpgl2.c getargs.c autogrph.c graphix.lib

Note that this package is compiled with the default optimizations.  Failure

use at least the default optimizations when compiling showhpgl.c may

result in a _TEXT segment which exceeds 64K and linking will fail.

graphix.lib is compiled in the small memory model.

Because graphix.lib internally uses Microsoft interrupt calling

functions, showhpgl must be compiled using the Microsoft compiler.  The object

module mscio.obj, which is in graphix.lib, contains all Microsoft, and PC

in general, specific functions.  This module must be replace in order to

use this package with other compilers and platforms.  Contact the author for

the required information.

Distribution Package

The distribution package includes the following:

showhpgl.doc		this document

showhpgl.exe		the executable program file

showhpgl.c		main program

hpgl1.c			additional functions

hpgl2.c			additional functions

hpgl.h			header file for showhpgl.c, hpgl1.c hpgl2.c

getargs.c		command line options and arguments

autogr.c		video graphix adapter detection

labels.c		labeling font for showhpgl (part of graphix.lib)

graphix.lib		graphics functions library

graphix.h		header file for main program (showhpgl.c)

graph.h			header file for additional modules (hpgl1.c,

hpgl2.c)

license.doc		license agreement

HP-GL test programs

aa_artst.dpf		AA, AR, CI, IP, SC, CT, PR: test general performance

of arc and circle drawing with fixed and adaptive

segment lengths

attest.dpf		AT: 3-point arc, absolute

actest.dpf		AC: anchor corner test

cptest.dpf		CP: character plot

ct_citst.dpf		CI, CT: circles with fixed and adaptive segment

lengths

di_drtst.dpf		DI, DR: absolute and relative label directions.

dv_sltst.dpf		SL, DV: test slant with DV command.

dvtest.dpf		DV: variable text path (fails this test)

ea_ertst.dpf		EA, ER: edge rectangle absolute and relative

estest.dpf		ES: extra inter-character spacing

ewtest.dpf		EW: outline wedge

ip_irtst.dpf		IP, IR: test & display usage of IP and IR

iwtest.dpf		IW: soft-clip window

labels.dpf		LB, LO, SL, SR, DV: labels and label origins, char.

slant, char. size (relative), direction vertical.

lttest.dpf		LT: line type: note that types 6-9 are identical as

types 7-9 are unsupported.

pa_prtst.dpf		PA, PR: test absolute and relative line drawing

petest.dpf		PE: polyline encoded

ra_rrtst.dpf		RA, RR: filled rectangle, absolute (no solid fills)

rttest.dpf		RT: 3-point arc, relative

sc_test.dpf		SC, DR, DI, LB: test all scaling types, DI, DR, and LB

instructions in normal and P1/P2 exchanged

coordinate systems.

smtest.dpf		SM: symbol mode

tl_lotst.dpf		LB,LO,TL,XT,YT: test labels, label origins, x-,

& y-ticks

fwdaltbx.dpf		drawing to demonstrate the -dp drawing option.

Use -C or larger for paper size.

Note: many of these program are dependent on using A-size paper to correctly

display the intended test functions.  Different results may occur from using

other paper sizes unless another paper size is specifically noted.

Some of these test programs are taken from "The HP-GL/2 Reference Guide"

and, as such are copyright 1990 by Hewlett-Packard.  This book provides

illustrations of what the output of some of these programs should look

like accompanying the description of the instruction.

The file labels.c is included for those who wish to modify the font used

for labeling.  This font is incorporated into graphix.lib.  To modify the

font, make the required changes to labels.c, compile it to a ".obj" file,

and update the library using "LIB -+labels;" (Microsoft library manager).

You may then re-link the remaining files to create a new program which

uses the new fonts.

Licensing

Please refer to the license agreement in license.doc.  Use of this

software and the enclosed source code is limited to the terms of the

license.

Bug Reports:

Please report any bugs to

becker@lennon.src.honeywell.com   (Robert Becker).

If you prefer, mail them to me at

Robert Becker

15836 Quebec Circle

Eden Prairie, MN   55346

Instructions Status

The following is a list of instructions and the level of implimentation

in showhpgl.exe of HP-GL/2 instructions.  Note that not all instructions

are HP-GL/2 instructions.  Those with notations in the source column

indicate where this instruction was found.  Status indicates the level of

implimentation of the instruction.  There are four levels of implimentation:

Full:		instruction is fully functional as per HP-GL/2

Partial:	instruction implimentation is incomplete

NOP:		no action or action not meaningful for this application

N/A:		not available: instruction was not implimented.

There are 38 Full, 8 partial, 26 NOP, and 29 N/A instructions.  Of the N/A

instructions, 11 are related to character set or font selection.  Of the

NOP instructions, 12 are related to output or digitizing a point and 4 are

for backward compatibility with earlier products.  All output instruction

that have no meaning in this application and are implimented as NOP

instructions.  No dual context mode instructions (HP-GL & PCL) have been

inplimented.

Command	Description			Status	source		Comments

AA	arc absolute			Full

AC	anchor corner			Full

AD	alternate font definition	N/A

AF	advance full page		NOP			compatibility w/HP9872

AH	advance half-page		NOP			compatibility w/HP9872

AR	arc relative			Full

AT	3 point arc absolute		Full

BP	begin plot			Partial			waits TWAIT seconds before clearing plot from screen

CA	designate alternate char. set	N/A	7470

CF	char. fill mode			N/A

CI	circle				Full

CP	character plot			Full

CS	designate std. character set	N/A

CR	color range			N/A

CT	chord tolerance			Full

DC	digitizer clear			NOP

DF	default				Full

DI	absolute label direction	Full

DL	download character		N/A

DP	digitize point			NOP

DR	relative label direction	Full

DS	designate char. set into slot	N/A	DraftPro

DT	Define label terminator		Full

DV	Define variable text path	Partial			No line feed reversal

EA	edge rectangle absolute		Full

EC	enable cutter			NOP

EP	edge polygon			N/A

ER	edge rectangle relative		Full

ES	extra space			Full

ESC	escape code instructions	Partial			No PCL mode switch commands

EW	edge wedge			Full

FI	primary font selection by ID	N/A

FN	secondary font selection by ID	N/A

FP	fill polygon			N/A

FR	frame advance			N/A

FT	fill type			Partial			Only hatched and cross-hatched fills

GM	graphics memory			NOP	DraftPro

IM	input mask			NOP	7470

IN	initialize			Full			Since there is no 'front panel', IN1 is same as IN

IP	input P1 & P2			Full

IR	input relative P1 & P2		Full

IW	input window			Full

LA	line attributes			NOP			No wide lines available

LB	label				Full

LO	label origin			Full

LT	line type			Partial			No adaptive line patterns: types 0-6, only.

MC	merge control			N/A

MG	message to panel		NOP

MT	media type			NOP

NP	number of pens			NOP

NR	not ready			Full

OA	output pen position & status	NOP	7470

OC	output commanded positon & sts	NOP	7470

OD	output digitized point & sts	NOP

OE	output error			NOP

OH	output hard-clip limits		NOP

OI	output identification		NOP

OO	output options			NOP	7470

OP	output P1 & P2			NOP

OS	output status			NOP

OW	output window			NOP	7470

PA	plot absolute			Full

PC	assign pen colors		N/A

PD	pen down			Full

PE	polyline encoded		Full

PG	page advance			Modified		Waits 6 seconds before clearing screen

PM	polygon mode			N/A

PR	plot relative			Full

PS	plot size			Full

PU	pen up				Full

PW	pen width			N/A

QL	quality level			NOP

RA	fill rectangle absolute		Partial			no user defined fills

RF	raster fill definition		N/A

RO	rotate coordinate system	N/A

RP	replot				N/A

RR	fill rectangle relative		Partial			no user defined fills

RT	relative arc 3-point		Full

SA	select alternate font		N/A

SB	scalable or bitmap font		N/A

SC	scale plot			Full

SD	standard font definition	N/A

SI	absolute char. size		Full

SL	char. slant			Full

SM	symbol mode			Full

SP	select pen			Full

SR	relative char. size		Full

SS	select standard char. set	N/A

ST	sort vectors			NOP

SV	screened vectors		N/A

TL	tick length			Full	7470

TD	transparent data		Partial			fn. char. set limited.

TR	transparency mode		N/A

UC	user defined character		N/A

UL	user defined line pattern	N/A

VA	adaptive pen velocity		NOP			compatibility w/HP9872

VN	normal pen velocity		NOP			compatibility w/HP9872

VS	velocity select			NOP

WG	fill wedge			Partial			Outline of wedge, only (no filling available)

WU	pen width unit selection	N/A

XT	x-tick				Full	7470

YT	y-tick				Full	7470

Notes:

BP instruction is checked for syntax.  BP will wait PG_DELAY seconds

before clearing the screen if the screen was plotted on.  If the

screen was not plotted on, the plotter is initialized.  In either

case, the plotter is initialized.

PG instruction generates a wait of PG_DELAY seconds before clearing

the screen.

PG_DELAY is defined in hpgl.h as 6 seconds.

HP-GL and HP-GL/2 distinguish between <cr> and <lf> for

positioning on labels.  The C-language, being based in UNIX,

assumes the <lf> is the newline character and <cr> has no

meaning.  This assumption is also present in this program and

some test case labeling and character plotting tests will fail

because of this.  The test program dvtest.dpf fails because of

this.  The display should look something like:

D A                  ABC

E B                  DEF

F C                     GHI

G

H

I

Several of the HP-GL/2 specifications are incomplete.  For

example, when one is drawing a wedge with symbol mode on,

placement of the symbol is unspecified.  Similarly, when TD

(transparent data) mode is on, the font used to display the

control characters is not specified.  Error handling is not

always specified.  For example, when CT is used to specify

deviation distance on circles and arcs, this reference does

not specify what action to take if a distance tolerance is

not specified for subsequent circles and arcs.  The DraftPro

manual specifies that the default shall be 5 degrees in this

case.  The Addison-Wesley book does not specify what action

to take.

For hatch and cross-hatch fills, it is not clearly specified

for the case of user-unit defined hatch x-spacing if the scale

in the y-direction should affect the spacing of the hatch. In this

implimentation, it was decided that the hatch spacing displayed on

the screen should be invariant with the rotation angle.

Additional questions arise in the case of the IR instruction.  One

would think that, for the case of P1 and P2 being opposite of their

normal locations (lower left, and upper right, respectively), that

the IR instruction would work off of these coordinates.  However,

there are indications of the HP-GL/2 reference guide that IR works

off of the default P1 and P2 locations.  That being the case, then

an IR of two arguments (IR x,y;) following an IP which exchanges

the P1 and P2 corners will take the x- and y-values as percentage

values based on the corners of convention for P1 and P2.  For

example, if one assumes IP10250,7479,250,279; and later executes an

IR 10,10;, for A-size paper, the IR corner plotter coordinates will

be (1030, 765), which, because P2 tracks P1 changes for only one

coordinate pair arguments to IR or IP, will force P2 to assume the

value of (-8970, -6534).  This is unlikely to be the assumed

result.  Further, executing an IR of four arguments

(IR x1, y2, xy, y2;) following an IP which exchanges P1 and P2

corners will have the effect of reverting the coordinate system to

the normal locations for the P1 and P2 corners

To use IR to exchange the P1 and P2 corners, the first coordinate

pair (x1, y2) must be greater than 50% (IR works in percent of the

hardclip limits), and the second pair (x2, y2) should be less than

50%.

HP-GL, HP-GL/2, HP, Hewlett-Packard, and DraftPro are registered

trademarks of the Hewlett-Packard company.

PC, CGA, EGA, VGA are or may be registered trademarks of the IBM

company.

UNIX is a trademark of AT&T.

Reference:

"The HP-GL/2 Reference Guide", Addison-Wesley Publishing Company.