MON.EXE is a monitoring utility to be used in conjunction with the

G8BPQ switch.  To date this is just a test version.  I expect MON

to become a much more useful network statistics collector in the

future.  Your ideas about what it should do are most welcome !

SETUP:

To setup and run mon.exe you will need to create a DESQview window

of about 40k.

On the DESQview Program line enter: c:\bbs\mon.exe

On the DESQview Parameters line enter: -m # (where # = bpq

stream number.)

The bpq stream number is a channel provided by the bpq host mode

and is not a com port number.  The suggested value is 32. However,

you may use any stream that is not being used by something else.

For Example:

If you are using YTNC with the parameters 25 1 1, bpq host

ports 1 thru 25 are already being used.	The first available

host port would be 26. (See YTNC.DOC for further information.)

The key here is NOT to use a port already in use for items like

PAC4 and CHAT4 which also use host ports.

On the DESQview Directory line enter: c:\bbs

The DESQview options and advanced options are set just like the mail

box (See Desqview in Install.doc)

Note: You may also call mon.exe from a dos window in DESQview.	From

The c:\bbs> prompt enter:

Mon -m 32 (or whatever stream number you desire. See above.)

Note: "m" is not a variable as in the bbs config files.

USE:

It has several commands.  The commands and capabilities are

still under development ... and subject to change.

enter	- command line parameters and features.

Command line format:

m

Port:

+#	- add channel # to display

-#	- drop channel from display

a	- monitors all ports

1	- monitors port 1, etc.

n	- monitors no ports

r		- toggles to monitor rx, tx, or rx and tx frames

i		- toggles to recieve text information or not.

l		- shows the number of calls heard recently.

Port: #	- enter port #

o file		- allows you to open a log file and log activity.

c		- closes log file opened with o command

q		- quit monitor

s		- stats command. Displays statistic on frames that MON has

seen on each channel, and the percentage of those bytes

that carry data with respect to the total bytes seen.

STATS COMMAND DISPLAY

FR	= Total frames heard of all types

%data = % of bytes that carry data

L3	= Layer 3 bytes total

L2	= Layer 2 bytes total

L1	= Layer 1 bytes total

REJ	= Number of FR frames that were rejects.

Note: MON cannot count retries, since it didn't hear them.  BPQ stats are

more accurate.

u	- toggles between showing all but UI frames in L list and all frames

in L list.