Copyright 1993 by Peter Sprenger   Pete@amber.dinoco.de

Muenchener Str.6

50170 Kerpen

Germany

Permission to use, copy, modify, and distribute this

software and its documentation for any purpose and without

fee is hereby granted, provided that the above copyright

notice appear in all copies.  The author Peter Sprenger

makes no representations about the suitability of this

software for any purpose.  It is provided "as is" without

express or implied warranty.

What is dmalib? Dmalib is an easy interface to initiate dma

transfers. Since one dma transfer can only occur in one dma

page, a big dma transfer has to be devided in a series of

subtransfers. When you use dmalib, it will initiate a transfer

up to the end of the used dma page. After the end of this transfer

(on most devices like soundcards there occurs an interrupt), you have

to call dma_next(). This routine will then initiate the next

subtransfer. After the end of this subtransfer you have to call

dma_next(), and so on...

The function dma_len() gives you the remaining bytes, that are to

transfer in subtransfers until the whole dma transfer is completed.

When this function gives you a value of zero, there is no need for

further calling of dma_next(). It will indicate the completion of the

whole dma transfer. The function dma_len() will show only correct

values BEFORE a subtransfer (before calling dma_next()).

--------------------  Documentation  ----------------------------

WORD dma_set(DWORD adr,DWORD len,int channel,BYTE mode);

The procedure dma_set() is used to initiate a big dma transfer. With

"big" a dma transfer that crosses dma pages is meant. The first

argument is the address where to start the dma transfer. The address

is the memory address as an unsigned long word. You can use values in

the range from 0 to 16mbyte (24 bits are used). The address is NOT in

segment representation (of course far ptr can only point up to 1mbyte).

The 2nd argument is the length of the dma transfer. The 3rd is the

dma channel number. The channel number is valid in the range 0 - 7.

The last argument specifies the dma mode. It is a combination of the

following:

[DMA_SINGLE | DMA_BLOCK | DMA_DEMAND] [DAM_IN|DMA_OUT] [DMA_AUTO] [DMA_DECREMENT]

An example for dma single transfer, out of ram and with auto

initializing:

WORD dma_set(0x8000,0x1000,1,DMA_SINGLE|DMA_OUT|DMA_AUTO);

The return value of dma_set() are the number of bytes that will

transferred in this subtransfer. For sound blaster programming for

example, you have to feed the DSP with this value.

WARNING: The dma channels over 3 are 16 bit channel (in an AT only).

Dma_set() will divide address and length through 2. So you can use

only EVEN addresses and an EVEN length on 16 bit channels. But this

enables you to access the range from 0 to 32mbyte and to make

128kbyte transfer (so say my docs - no warranty!).

WORD dma_next(int channel);

Just call dma_next() with the dma channel number, and the subtransfer

is intiated. The return value is like dma_set() the number of bytes

that will be transferred in the next subtransfer.

DWORD far2long(char far *adr);

The function far2long takes a segmented far (or huge) pointer and

returns a 32 bit address, that can be used for dma_set().

long dma_len(int channel);

This function gives you the number of bytes that are to transfer to

complete the whole dma transfer. The normal purpose of dma_len is the

calling to determine if there are subsequent calls of dma_next()

needed.

-----------------------------------------------------------

Vers.                          History

-------

1.00 20-Jun-93  dmalib 1.0 library released