IMP

(NOTES ON HOW TO USE)

Copyright 1985, 1987

Irvin M. Hoff W6FFC

01 June 1987

INTRODUCTION:

------------

NOTE:  THIS PROGRAM SUPPORTS ANY 1200 OR

2400 BPS MODEM INTENDED FOR AUTO-

DIALING WITH THE 'AT' PROTOCOL OR

THE RACAL-VADIC PROTOCOL. IT ALSO

SUPPORTS THE  PROMETHEUS PROMODEM

EXTENDED-'AT' 1200 BPS PROTOCOL.

NOTE:	IMP v245 adds break capability.  Use ESC Q.

It also adds the 'TIG' toggle command to

display all incoming control characters if

needed for a remote editor.

The IMP modem program enables a computer to communicate with

another computer.  This is normally done by using standard telephone

lines but with special audio tones sending serial data.  Three common

speeds are used, depending on the facilities available -- 300, 1200 or

2400 bps.  ('bps' stands for bits per second.  One character is 10 bits

normally.  It may be 11 bits if two stop pulses are used.)

WHAT IS A MODEM?

---------------

A "modem" is provides these audio tones.  This is an acronym for

"modulator and demodulator".  The one section (modulator) provides the

audio tones for sending over the telephone line and the other section

(demodulator) then converts those audio tones back to digital pulses the

computer uses to receive the incoming data.

The modem itself is a hardware utility, usually costing around

$50 to $125 for a typical 300 baud device and from $80 to $250 for a

system offering 1200 bps as well as 300 baud.	The newest modems add

2400 bps and currently can be obtained from $200 to "way on up".  (The

so-called "list prices" are rarely paid even with single-unit purchases.)

Some modems are intended for plugging directly into the computer

while a vast majority used with CP/M computers are external devices that

attach to the computer through a serial I/O connector.	These are often

designed to sit under the phone itself to minimize the space required.

An external modem has several advantages as most have numerous indicator

lamps telling the user what is happening.  This also makes it far more

simple to use the modem on any of several computers the user may have.

(It also makes it a lot easier to adjust the loudspeaker volume control

than on an internal unit.)

AUTO-STEPDOWN, A NEW MODEM FEATURE

--------------------------------

The newer 300/1200/2400 bps intelligent modems have the ability

to determine the maximum speed (baudrate) at which the system they are

calling can use.  They adjust themselves accordingly, without operator

intervention or knowledge.  They send a "result code" back to the local

computer telling it what speed they now have.  This information can be

used to automatically set the local computer to the same speed.  A mes-

sage is displayed telling the operator what speed he has.

If the operator has a reason for using less than the maximum

speed to which another system might respond, he can initially restrict

the modem from attempting to connect at a faster speed.  Example, Compu-

serve has modems that will answer at either 1200 or 300 bps.  If reading

and answering messages "on line in real time" many callers prefer to use

300 baud (they cannot type even that fast) rather than 1200 bps which is

nearly twice the cost.	(They also have 2400 bps modems but those are

frequently on an independent phone line not used for 300 or 1200 bps.)

IMP takes advantage of this automatic stepdown feature.  It has

the modem attempt to connect at 2400 bps.  If that is unavailable at the

answering end, the modem tries for 1200 bps.  When connected, IMP shows

the speed currently in use and automatically adjusts the local I/O to

match.	(IMP is the first and currently the only free modem program to

take advantage of this new modem feature.)

The operator can easily select the "SET" command should he wish

to override the automatic "start trying at 2400" and have a specific

speed dialed (such as 300 baud for Compuserve for hand typing messages

at lower cost, etc.)  The automatic feature is reinstated by either

using the "SET" command for 2400, or merely calling up the modem program

from the disk once more.

Note: The US Robotics Courier auto-steps from 2400 to 1200 but

not to 300, as do most of the other new modems with 2400 bps.  This is

no loss, really, as few people using 2400 would call into a system only

having 300 bps.

PURPOSE

-------

The program discussed in this guide allows each computer involved

to directly control its own modem, converting DC pulses into audio tones

which are sent to (or received from) the telephone line.  (The standard

1200 baud transmissions use "DPSK" (dibit phase shift keying) instead of

"AFSK" (audio frequency shift keying).

For short distances, no modems are required as the computer can

provide the necessary digital voltages.  This discussion is centered

around systems intended for use over distances ranging from several

miles to literally around the world.

FEATURES:

--------

A program to communicate with another computer can range from very

simple to very complex.  Although IMP has a number of quite useful fea-

tures (some of which other modem programs do not currently offer) it is

one of the easier programs to learn and use.  A list of some of the

features offered by this particular program include:

1)  Ability to talk with another computer via keyboard at each end.

(Called the "terminal mode".)

2)  Ability to operate a remote mainframe computer such as TYM-SHAR,

or special data bank systems now commonplace.  This includes

"bulletin board" systems for receiving, sending or just read-

ing messges or other types of information.

3)  Ability to upload (send) or download (receive) programs from other

computers whether TYM-SHAR, data banks, remote hobby systems

or just another individual in another location.

4)  Ability to automatically transfer a group of files (called "batch

mode".  Two popular methods included, KMD batch mode and the

older MODEM7 batch mode.  (Only the KMD type is supported by

RCPM or RAS systems, the MODEM7 batch type can be used when

in contact with another individual that only has the MODEM7

type.)

5)  Ability to copy incoming information on a printer.  If the print-

er is too slow to copy the data directly, the extra characters

back into a large storage buffer until they can be printed.

6)  Ability to copy incoming data into memory for automatic transfer

to a disk file.

7)  Accomodates any computer clock speed up to 25.5 MHz. in 0.1 Mhz.

increments for uniform results.  (The clock speed is actually

of modest interest, it is only used for setting timing loops.)

8)  Advanced type of error detection (CRC or Cyclic Redundancy Check)

using automatically generated high-speed lookup tables.

9)  Ability to send any of 10 different pre-programmed function keys.

(These keys are very easy to reset, using either of two exter-

programs provided.)  An 11th key is available for log-on use.

11)  Ability to send a 'break tone' to control mainframe reset and for

use with PC Pursuit to disable the '<ret> @ <ret>' disconnect.

This uses ESC Q, can be set otherwise in the overlay.

10)  Automatic disconnect from the telephone line when using ESC N, "D"

or "Z" as an option during file transfer.

11)  Automatic or manual 1k protocol for increased throughput and effi-

ciency over the 128 character blocks long associated with such

programs.  (Effectively gains 10% at 1200 bps and 20% if using

2400 bps.)

12)  Supports 1200 or 2400 bps modems using the Racal-Vadic or 'AT'

protocol developed by Hayes.	These features basically include

automatic dialing and redialing until the system being called

finally answers.

a) Automatic selection of a particular phone number from

a library of names and numbers.  (These phone numbers

can be very easily changed by using either of two ex-

ternal programs that are provided.)

b) Automatic dialing for alternate long-distance systems

(where auto-dialing touch-tone modems are used).

c) Entry of a hand-typed number.  Although it might be as

simple to manually dial the number as hand-type it, it

can then be automatically redialed indefinitely.

d) Auto-linking of 1-32 different numbers for continuous

dialing until one of the group is available.

e) Choice of beeping continuously when connected to get

the operator's attention, or beep one time and go to

terminal mode immediately to catch UNIX or IBM signon

messages that assume you can copy when connected.

f) Automatically adjusts the terminal speed to that of

system answering the phone.

USING THE PROGRAM THE FIRST TIME:

--------------------------------

For now, we shall assume the program is ready to use on your equip-

ment.  (Installing the appropriate overlay to achieve this result will

be discussed later.)

IMP v245 modem pgm (type M for Menu)

Copyright (c) 1985, 1987 Irvin M. Hoff

Version for Datapoint 1560 modem port 28H

Modem speed is: 2400 bps

B>>COMMAND: x

(Your version would look different at line 3 once the

appropriate overlay was added and the baudrate might

show 1200 if you do not have 2400 bps available.)

The cursor stops at point "x:" waiting for some sort of command.

Typing a "M" followed immediately by a RETURN, shows the first page of

a 3-page help guide.

Several commands are similar to those already used for CP/M, such

as 'ERA', 'DIR' and the ability to change disk drives and user areas.

This adds tremendous flexibility while remaining in the modem command

mode.  To change disk drives, just say:

B>>COMMAND: A: <ret>

and you are in the same user area on the A: drive.  You can also

say:

A>>COMMAND: A12: <ret>

to change user areas.  When finished (using this example), say:

A12>>COMMAND: CPM  <ret>

or

A12>>COMMAND: BYE  <ret>

and you will go back to CP/M with the original drive and user area.

OPTIONS:

-------

There are 6 primary options and 9 secondary options that may be

typed on the command line.  In addition there are other numerous commands

ranging from single character to three characters that can be used (such

as DIR, ERA, LOG, etc.)  Those are listed on the menu.

Primary Options:	 E, L, M, R, S, T

E = Echo mode (special type of full duplex)

L = Local modem (no remote echo, half duplex)

M = Menu display showing features that are available

R = Receive a file from remote facility

S = Send a file to a remote facility

T = Terminal mode (full duplex, echo provived remotely)

Secondary Options:  A, B, D, J, O, Q, R, S, V, X, Z

B = Batch mode -- allows automatic multiple file transfer

Most RCPM systems now use KMD in place of the older

XMODEM program.  KMD offers YMODEM-type batch mode.

This can also simplfy single-file transfers since

the filename only needs to be typed at one end.  It

also permits using "wildcards" which makes it easier

to type filenames.  Batch mode is also useful from

modem-to-modem (as when talking to a friend, etc.).

D = Disconnect when done, stay in IMP

K = Manually initiates 1k protocol in send mode

Q = Quiet mode, special purpose feature rarely used except if

operating through a "BYE" program.

R = Receive remote file (8-bit binary)

S = Send file to remote system (8-bit binary)

V = Visual inspection of an ASCII file being transferred.  Not

often used as most ASCII files are now "crunched" for

minimum transfer time and disk space.

X = Inhibits automatic 1k request, transfers 128 char. blocks

Z = Exit to CP/M  when finished, disconnect from phone line

Although you can get the following information from the help guide

in IMP itself, showing it here will allow us to discuss the various fea-

tures in somewhat better detail.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

THE HELP GUIDE, PAGE 1:

----------------------

Single Letter Commands

?  - Display current settings

ESC - Function key intercept character, then (0-9)

M  - Display the menu

E  - Terminal mode with echo

L  - Terminal mode with local echo

T  - Terminal mode

^Z  - Clears screen (command mode only)

R  - Receive an 8-bit binary CP/M file

S  - Send an 8-bit binary CP/M file

COMMAND: R (or S) FILENAME.TYP

R and S can use the following subcommands:

B  - Batch transfer, can use wildcards (e.g., *.*)

D  - Disconnect when done, return to command mode

K  - Manual request for 1k transmissions

Q  - Quiet mode (no messages to console)

V  - View ASCII files on CRT during a file transfer

X  - Inhibits auto 1k request to sender

Z  - When done, disconnect, go to CP/M

For copying text to disk use T (E or L) FILENAME.TYP

[more]

(The single letter commands may also be used on the

command line when the program is initially executed.)

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

DISCUSSION OF HELP GUIDE PAGE 1:

-------------------------------

There are basically three types of commands that may be used with

this program:

1)  Single-letter commands shown on page one.

2)  Three-letter commands shown on page 2, and

3)  Control-<char.> commands shown on page 3.

Going down the list:

? - shows current parameters.  As these are closely tied in with some

of the other options, we shall show a typical example later

in this discussion.

ESC - shows the function key intercept character.  This may be easily

changed by selecting a different character at the 'EXTCHR'

option in the overlay.  (Byte 0114h.)

NOTE:  typing ESC <ret> while in the command mode will display

all the programmable function key definitions with the

exception of the log-on string which uses ESC L.

M - shows the menu a page at a time.	You may abort after any page is

shown, with a CTL-C.

E - terminal mode with echo.	When talking with another terminal also

also using IMP (or a comparable program), one of you must send

an 'echo' back so you can both see what the other person is

typing.  This is needed for full-duplex operation.  Usually

the person originating the call will type "E" instead of "T".

However either may do this.  If "both" use the echo mode, the

program usually starts typing a string of feedback characters

since both computers are re-sending the same character over

and over.  You can stop this by just returning to command mode

(type control-E) and then retyping "T" rather than "E".

L - terminal mode with local echo.  Shows what you are typing but does

not send an echo of the other person's typing back to him.

Can be used at both ends, in place of one person using "E".

Both "E" and "L" are useful at times for some types of modems

that do not use full-duplex, or for some main frame units that

run half-duplex.  The combination of "T", "E" and "L" gives an

excellent choice to use on various systems.  Normally the "T"

mode is sufficient except when talking with another person and

then the "E" mode at one end or "L" mode at both ends will

provide normal results.  If the remote system does not supply

a LF after a CR, you can get that from the 'TLF' command.

T - puts the program directly into "terminal mode", allowing it to

send from the keyboard to the other computer, and to copy any-

thing coming from the other computer with the exception of

file transfers (use "R" to receive those).

R - used only for receiving files from the other computer.  These are

usually sent in binary form with either checksum verification

or more commonly, CRC (Cyclic Redundancy Checking).  If ready

to receive another file, you have several choices, depending

on the program used by the other computer.   After notifying

him you are ready (or will be very shortly), you can type:

B>>COMMAND: R HELLO.DOC <ret>

and when he starts to send, you will receive the program which

then goes directly to the disk.  You could also have said:

B>>COMMAND: R <ret>	or   RB <ret>  if you prefer

This is called the "Batch mode" and is an extremely useful way

to receive programs automatically, from another computer using

a similar program.  He will use the batch mode for sending the

programs and that automatically provides the file names at the

receiving end.  In this manner, large numbers of programs can

be transferred completely automatically, including full disks.

(Assuming you care to pay the toll calls for the time it will

take!)

NOTE:  The method used to receive batch mode would be:

B>>COMMAND: R <ret>    (also RB <ret> may be used)

since you may not know in advance how many programs he

will be sending.

NOTE:  While receiving a file, any errors will be shown

on the console CRT, indicating that portion of the

program, if being repeated.  If the file finishes

transferring normally, it will have been received

error-free.  Those error notices are merely to let

the user know a momentary problem caused a resend.

The error messages indicate phone line quality.

S - used only to send programs.  Similar to "R" but sends instead of

receives.  If the other system is ready to receive, you could

then type:

B>>COMMAND: S HELLO.DOC <ret>

It will tell you how many records are involved and how many

minutes and seconds it will take to send the program.  If the

other system is ready to receive, it starts the transfer,

showing how many records have been sent.  If any errors occur

during transmission, it shows what they are and counts them.

It will resend any records not correctly received at the other

computer.  If too many consecutive errors occur (normally set

at 10), the transfer is terminated.  The 'batch mode' may also

be used for sending:

B>>COMMAND: SB HELLO.*		 (sending batch mode)

will send all files starting with HELLO, or can send just one

file, or can send several non-related files in this manner:

B>>COMMAND: SB HELLO.DOC XDIR.COM TERM.*

etc.	Be sure to put a space between the various files.  You

can include the 'Z' option:

B>>COMMAND: SBZ HELLO.* IMP740.LBR I2LIB.COM

When the file transfer is completed it disconnects from the

phone line and reverts to CP/M.  This allows you to be absent

when the file is finished, offering an advantage when sending

lengthy files.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

THE HELP GUIDE, PAGE 2:

----------------------

Three Letter Commands

BYE - Disconnect, then return to CP/M

CAL - Dial number

CPM - Exit from this program to CP/M

DIR - List directory and space free (may specify drive)

DSC - Disconnect from the phone line

ERA - Erase file (may specify drive)

LOG - Change default drive/user no. (specify drive/user)

and reset disks.	e.g. LOG A0: or LOG B: (user # unchanged)

SET - Set modem baud rate

SPD - Set file output speed in terminal mode

TBM - Toggle MODEM7/KMD batch mode selection

TCC - Toggle CRC/Checksum mode on receive

TIG - Toggle 'ignore CTL character' on/off

TLF - Toggle LF after CR in "L" or "T" mode for a disk file

TRB - Toggle rubout to backspace conversion

The following are terminal text buffer commands:

DEL - Delete memory buffer and file

WRT - Write memory buffer to disk file

[more]

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

DISCUSSION OF HELP GUIDE, PAGE 2:

--------------------------------

BYE - Disconnects the modem from the phone line, closes any file that

may be open and returns to CP/M.

CAL - Displays the telephone library.

NOTE:	Use the phone number overlay I2NM-2 to easily change the

numbers that are stored.  This overlay also accesses up

to two alternate long distance dialing systems for the

touch-tone auto-dialing modems.  You can also use an ex-

ternal program called I2LIB to rapidly change any phone

numbers.

CPM - This returns to normal CP/M use, abandoning IMP.

CAUTION: It does not disconnect the modem from

the phone line, which allows intentional

return to CP/M temporarily.

DIR - Enables the user to see the library of any disk without needing

to return to CP/M first.  Shows the amount of disk spacing

remaining on the logged-in default drive.  (You must log in

any drive you wish to write on, or to see the correct free

disk space remaining.)  See the LOG feature below.

DSC - Disconnects the modem from the phone line whenever selected.

ERA - Enables the user to erase any files from any disk without the

need to return to CP/M first.  Wild cards (* and ?) accepted.

LOG - When a new disk is inserted, you can see the directory and read

files from it but cannot write to it without getting a "BDOS

ERR".  This command allows you to log in a new disk so you can

immediately write on it without first returning to CP/M for a

warm boot.  You can select drive and user number.  (This also

enables DIR to show the correct free disk space remaining.)

You can say:

LOG <ret>  for same drive, user area, new disk

LOG A

LOG A:

LOG A14    (user areas are 0-15)

LOG A11:

SET - Allows the user to change baudrates when desired.  If using a

2400 bps modem with automatic stepdown, this would rarely be

used.

SPD - Sets the delay between characters and/or delay between lines in

the "T" mode when using ESC F upload a pre-typed file.  Since

most bulletin boards and TYM-SHAR systems normally use line

editors that require a delay after they have received a line,

you can add delay between characters (in increments of 4 ms.)

and/or enter delay between lines (in increments of 100 ms.)

This is preset for most systems at 2 and 200 ms.  They can be

changed at any time.	Current values can be checked by using

the  " ? " command.

TBM - Toggle batch mode from KMD (default) to MODEM7.  The primary

difference is the way the file name is sent.	The older MODEM7

method sends each character separately, expecting the other

computer to echo each character.  Checksum is used to verify

the file name.  The KMD method sends a special 128 byte block

with the file name and the length of the file.  This is not

only far better for use with mainframe and networking systems,

but also informs the operator receiving the file how long it

will be.  (The KMD method then automatically goes to 1k mode.)

The chaces are excellent you will never need the MODEM7 type.

TCC - Most modem programs originally used simple checksum verification

of received sectors.	Most now use Cyclic Redundancy Checking

which finds certain transmission errors the simple checksum

system might overlook.  This toggle allows the user to select

checksum for those systems known to not offer CRC.  When set

normally to CRC, the program automatically switches to check-

sum after trying several times to use CRC.

TIG - Toggles the 'IGNRCTL' flag in the overlay.  This is usually set

to ignore incoming control characters other than those needed,

such as CR, LF, backspace, etc.  This minimizes unwanted char-

acters sometimes received in noisy conditions which can create

undesired effects.  Can be toggled to receive ALL incoming CTL

characters for the occasional use of an editor requiring them.

TLF - If sending a file while in terminal mode (via ESC F option) most

mainframe computers will not want you to send a line feed.  As

all editors include the LF after CR, you have the option with

this toggle of sending or not sending a line feed.  If typing

manually in the "T" mode, you can get a line feed with CTL-J.

It is automatically included after CR with "L" or "E" modes.

TRB - Allows the rubout char. to be changed to backspace if desired.

Can be set in overlay to default either way you prefer.  Set

to 'backspace' in distributed overlay copies.

DEL - If copying incoming information into memory, and you decide you

do not want to bother saving it to disk after all, this closes

(and then deletes) the disk file you had opened.

WRT - saves what you have been copying to the disk file already opened

for that purpose.  When returning from the "T" mode to command

mode, a warning is shown that you may lose what has been saved

unless you use WRT (write to disk).

NOTE: An open file is automatically written to disk

should you use BYE or CPM commands to leave the

program.  This prevents accidental loss of data

intended to be saved.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

THE HELP GUIDE, PAGE 3:

----------------------

Local Commands while in Terminal Mode

ESC E  - Exit to command mode

ESC F  - Send file to remote system

ESC L  - Send log-on message

ESC N  - Disconnect from the phone line

ESC P  - Toggle printer on (or off)

ESC Q  - Send break tone

ESC Z  - Clears screen, terminal mode

ESC R  - Stop copy into buffer

ESC Y  - Start copy into buffer

Start & Stop may be toggled as often as desired.

A ";" at start of line indicates buffer is copying.

X-off automatically used to stop input when writing

full buffer to disk, X-on sent to resume.

>>COMMAND:

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

DISCUSSION OF HELP GUIDE, PAGE 3:

--------------------------------

ESC E  - Returns to command modem from "T", "L", "E" modes.

ESC F  - Sends an ASCII file to the remote terminal, X-on and X-off

automatically used.  Optional delays available.

ESC L  - Will send the log-on message if one has been included in the

LOGON area of the overlay.  (This is intended only to send

a phrase of ASCII characters, not intended as a substitute

for "script" commands.)

ESC N  - Disconnect the modem from the phone line.

(You can easily change this to whatever you like in the

overlay you use for your equipment.)

ESC Q  - Sends a 300 ms. break tone.	Sometimes required by mainframe

computers to reset.  Used by PC Pursuit to disable the

'<ret> @ <ret>' disconnect command, etc.

ESC P  - Toggles the printer on-off.	Very useful feature.  Similar to

normal control-P but in this case if the modem is running

faster than normal printer speed, the characters just back

into a special buffer while the printer tries to catch up.

Especially beneficial when using 2400 bps as few printers

can keep up at that speed.  Can be toggled at any time you

are in the "T", "L" or "E" modes.	Often used to copy items

from bulletin boards, etc. that are not worth permanently

saving on disk.

Note:	Some computers, such as the Commodore

Commodore C128 cannot use the printer

and the modem at the same time.

ESC Z  - Clears the screen in terminal mode.	(CTL-Z normally used but

that goes on out the terminal to the remote system.)

ESC Y  - Used only in terminal mode.	If ESC R is used to temporarily

stop copying incoming text to memory, ESC Y restarts this

feature.  When copying to memory, a ';' appears in the

first column of each line.

NOTE:  To open a file use:  COMMAND: T FILENAME.EXT <ret> and

then ESC R and ESC Y will control what is copied for

saving to disk.

ESC R  - Stops the copy into memory, shuts off the ";" at start of

each line.  These two commands (ESC Y to start, and ESC R

to stop) control what is being saved.  When returning to

command (via ESC E), you will then be warned to save the

information to disk via the WRT command or it will likely

be lost.  (You may never use ESC R or ESC Y and just use

your editor on that file later to discard unwated magerial.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

AUTO-DIALING AND AUTO-REDIALING:

----------------------------------------------------------------

There are several ways to auto-dial.

1) from the command line

2) from the phone number library line

In addition you can auto-redial from either of these lines.  Once

the number has auto-dialed and there is no answer, it will ask you if

you want it to continuously auto-redial until it does receive an answer

(or you decide to abort).

To get the phone library, type CAL <ret> on the command line.  You

then normally just type the appropriate character at the start of the

line.  It will show the number being dialed.  You can also do this from

the main command line if you know in advance what character is used for

that particular number:

B>>COMMAND: CAL P <ret>

This calls up the "P" line in the library and dials that number,

then if busy asks if you want it to continue redialing.  You can use:

B>>COMMAND: CAL 313-749-6569 <ret>

and it will dial the number specified.  You can then do either of

these on the library command line, as well.  This gives a tremendous

versatility.

NOTE:	To access the alternate long distance dialing systems (up to two

are offered), use the phone number overlay and fill in the in-

formation regarding your access number, billing number, etc.  It

will then show how to enter a special character ahead of each

phone number where alternate long distance system is desired.

This only works where touch-tone systems are in use.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

AUTO-LINKING PHONE NUMBERS:

--------------------------

If the number you want to dial is busy, you would like to have it

automatically redialed until it answers.  However, you might like to

call any of a group of numbers and be happy to take whichever one first

becomes available.  Auto-linking allows a group of numbers to be listed

and they are then continuously dialed one after the other, until one is

available.

IMP permits from 1-32 numbers to be linked together in any manner.

There are two options:

B>>COMMAND: CAL A,D,R,3,Z	     1)

B>>COMMAND: CAL F,		     2)

B>>COMMAND: CAL A/D/R/3/Z	     3)

B>>COMMAND: CAL F/		     4)

B>>COMMAND: CAL A,D,A,R,A,3,A,Z   5)

B>>COMMAND: CAL F		     6)

1) using a comma (,) will beep continuously when connected to

the first available number - touching any key stops the beep

and goes to terminal mode.

2) just the one number is linked to itself and redials until

connected.  Beeps continuously until stopped with any key.

3) using a slash (/) will beep only once when connected and then

automatically go directly to terminal mode - this is useful

where some mainframes such as IBM and UNIX send a logon note

assuming you are already able to copy as soon as connected.

4) just the one number is linked to itself and redials until

connected.  Beeps only once and goes to terminal mode.

5) illustrates how one number can be given top priority.

6) system used previously - if connected on first try, beeps one

time and jumps directly to terminal mode.  If no connection,

asks the operator if he wants to try again or try continuous-

ly until a connected is made - at which time it beeps until a

key is depressed.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

CASCADING COMMANDS:

------------------

Several commands may be used on the same line.  Ones normally used

are:  B, D, E, K, L, Q, R, S, T, V, X and Z

The commands have been listed previously but will be listed here

again:

B) batch mode  (must be used at both ends if used at all)

(usually only available modem-to-modem with operators

present at each end.)

D) disconnets the phone, returns to CP/M after a file transfer

E) echo mode for conversing with a system not offering echo.

K) used to manually request 1k transmissions on systems not

having automatic 1k transmissions.

L) local echo shows what you are sending if no remote echo.

Q) quiet mode, suppresses messages on local console.

R) receive file transfers (8-bit binary)

S) send file transfers, also shows transfer time.

T) terminal mode - operates computer at other end like a remote

terminal.  Also stands for "text mode".  When in "T" mode

you can copy incoming to printer or to disk, and send pre-

typed ASCII text files via ESC F.

V) visual -- lets you see incoming ASCII files while being sent

to the disk system.

X) inhibits automatic request for 1k transmissions

Z) disconnects the phone, returns to command mode after a file

tranfer.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

TYPICAL PARAMETERS SHOWN BY THE "?" COMMAND:

-------------------------------------------

B>>COMMAND: ? <ret>

Mode: CRC

KMD batch mode

Rub is backspace

Printer buffer is OFF

Modem speed is: 1200 bps

Incoming control characters received

Terminal mode file buffer is inactive

Unused portion of buffer is 16384 bytes

Use ESC before local command in terminal mode

XOFF testing used in terminal mode file output

LF NOT sent after CR in "L" or "T" for a disk file

Char. delay (terminal file mode) is:   2 ms. per character

Line  delay (terminal file mode) is: 200 ms. per character

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

DISCUSSION OF TYPICAL PARMETERS SHOWN BY THE "?" COMMAND:

--------------------------------------------------------

By this time, most of these parameters are self-explanatory.  They

primarily show how the options and toggles have been set.  The user can

check on any of the major parameters at any time by:

1) Typing ESC E to return to COMMAND: mode and

2) Typing  ?	to see the list of current parameters.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

FUNCTION KEYS:

-------------

There are 10 function keys which may be easily programmed with an

external file called I2FUNC.COM in just a few moments.	It makes the

changes directly to the IMP.COM file.  (There is an additional ESC L

log-on selection that is programmable in your overlay.)

To view the contents of the function keys, type 'M' for menu while

on the command line and it will show the character currently used for

the function key intercept.  (It will be ESC unless you select another.)

You could then type that character (followed by <ret>) on the command

line and it will display the entire contents of the function keys.

NOTE:	ESC L gives the string of characters set

into the 'LOGON:' area of the overlay.

This can be used for a variety of uses.

You can readily program that for whatever

purpose you wish.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

CHANGING THE PHONE NUMBER LIBRARY:

---------------------------------

The phone numbers in the library may be easily changed using an

external file called I2LIB.COM.  This permits any of the 36 phone num-

bers to be changed in only a few seconds.  It works directly on the

IMP.COM file.  If extensive changes are to be made, you may prefer

to use an overlay file called I2NM-2.ASM.  This permits using your

editor to make quick and selective changes, or to substitute an entire

new library.  (You may need to add or remove things like '1-' for long

distance dialing in your area.)  That file also is used to set the

alternate long-distance dialing routines for SPRINT, MCI, TYMNET, etc.

It can also be used for changing the function key assignments.	(You

can even imbed control characters into function key assignments using

the I2NM-2.ASM file.)

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

RETRY OPTION:

------------

The consecutive error limit has been set to 10.  When transferring

files, if this limit is reached, the file transfer will be aborted.

-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -	-   -

INSTALLING THE PROGRAM:

----------------------

Although this is obviously the very first thing that needs to be

done, it was saved until last for several reasons.  The most important

of these was the need to first explain what some of the options would

do when selected.

By this time the user should have a fairly good idea what options

would work best for him.  When looking at the beginning of the program

where the these are selected, it should now be rather easy to pick the

ones that might need to be changed for a particular preference.  You may

be using a 2400 bps modem and find the overlay was set for 1200, etc.

There are various ways to change the program to suit a specific

user.  You are offered several methods.  The "easy way" is to get the

IMP.COM file plus the appropriate overlay for your computer.  You then

edit the overlay (if needed) to suit your local needs, assemble it to

get a .HEX file then use the simple instruction and MLOAD.COM to combine

the standard IMP.COM file with your overlay.  The front end of each

overlay contains all the information on how to accomlish this.

Generally no changes need to be made in specific overlays unless

you have some particular requirement different from others using the

same equipment.  (Such as the log-on message, for example.)

You can also use DDT to change specific bytes without editing and

assembling.

NOTE:	Although designed with special emphasis on the auto-

dialing and redialing capability of 1200 or 2400 bps

modems using the Racal-Vadic or Hayes 'AT' protocol,

the program may be used with other modems where the

operator manually dials the number, then changes to

terminal mode after connecting his modem.

SUMMARY

-------

We hope this verstile program will give you hours of enjoyment and

fulfill your modem needs.  It is particularly useful with the 2400 bps

modems that have automatic stepdown.

- Irv Hoff W6FFC

(415) 948-2513 PRACSA RCPM

(415) 948-2166 voice

######################################################################