Rancho Technology, Inc.

10783 Bell Court

Rancho Cucamonga, CA. 91730

Dear Valued Customer,

The Rancho RT1000A/B-OS/2 2.0 Disk Driver was written to allow the RT1000A/B

host adapter to support SCSI hard disks and CDROMs under the IBM OS/2 2.0

operating system. The Driver currently supports one RT1000A or RT1000B host

adapter in a system.

The minimum hardware requirements are:

* AT computer w/an Intel 80286-compatible CPU (i.e. 286/386/486).

* 40-Mbyte SCSI hard disk.

* 4-Mbyte RAM.

* VGA color monitor (required for OS/2).

* RT1000A/B SCSI host adapter w/RTBios V8.20R or later.

The minimum software requirements are:

* IBM OS/2 2.0 operating system.

* RT1000A/B-OS/2 2.0 Disk Driver Diskette.

* DOS 3.3 or higher operating system (optional).

The Rancho RT1000A/B-OS/2 2.0 Disk Driver can be installed immediately following

the installation of the IBM OS/2 2.0 operating system or any time thereafter.

Follow the steps below for proper installation:

Step 1:	Go to the OS/2 Window.

Step 2:	Copy the Rancho OS/2 Disk Driver, called RT1000.ADD, to the root

directory of the hard disk.

Step 3:	Use any editor to add the following entry to the CONFIG.SYS file:

BASEDEV=RT1000.ADD

Step 4:	Shut down, reboot the system, and you will have completed installing

the Rancho RT1000A/B-OS/2 2.0 Disk Driver!